BASIC ASSESSMENT REPORT

And

ENVIRONMENTAL MANAGEMENT PROGRAMME REPORT

SUBMITTED FOR ENVIRONMENTAL AUTHORIZATIONS IN TERMS OF THE NATIONAL ENVIRONMENTAL MANAGEMENT ACT, 1998 AND THE NATIONAL ENVIRONMENTAL MANAGEMENT WASTE ACT, 2008 IN RESPECT OF LISTED ACTIVITIES THAT HAVE BEEN TRIGGERED BY APPLICATIONS IN TERMS OF THE MINERAL AND PETROLEUM RESOURCES DEVELOPMENT ACT, 2002 (MPRDA) (AS AMENDED).

NAME OF APPLICANT: Raubex Construction (Pty) Ltd

TEL NO: 072 591 8878

FAX NO: -

POSTAL ADDRESS: -

PHYSICAL ADDRESS: Building 1, High Grove Park, 50 Tegel Avenue, Highveld 0169

FILE REFERENCE NUMBER SAMRAD: GP30/5/1/3/2/10278MP

- IMPORTANT NOTICE

In terms of the Mineral and Petroleum Resources Development Act (Act 28 of 2002 as amended), the Minister must grant a prospecting or mining right if among others the mining “will not result in unacceptable pollution, ecological degradation or damage to the environment”.

Unless an Environmental Authorisation can be granted following the evaluation of an Environmental Impact Assessment and an Environmental Management Programme report in terms of the National Environmental Management Act (Act 107 of 1998) (NEMA), it cannot be concluded that the said activities will not result in unacceptable pollution, ecological degradation or damage to the environment.

In terms of section 16(3)(b) of the EIA Regulations, 2014, any report submitted as part of  an  application  must  be  prepared  in  a format  that  may  be  determined  by  the Competent Authority and in terms of section 17 (1) (c) the competent Authority must check whether the application has taken into account any minimum requirements applicable or instructions or guidance  provided by the competent authority to the submission of applications.

It is therefore an instruction that the prescribed report required in respect of applications for an environmental authorisation for listed activities triggered by an application for a right or a permit  are submitted in the exact format of, and provide all the information required in terms of, this template. Furthermore please be advised that failure to submit the information required in the format provided in this template will be regarded as a failure to meet the requirements of the Regulation and will lead to the Environmental Authorisation being refused.

It is furthermore an instruction that the Environmental Assessment Practitioner must process and interpret his/her research and analysis and use the findings thereof to compile the information required herein. (Unprocessed supporting information may be attached as appendices). The EAP must ensure that the information required is placed correctly in the relevant sections of the Report, in the order, and under the provided headings as set out below, and ensure that the report is not cluttered with un- interpreted information and that it unambiguously represents the interpretation of the applicant.

- Objective of the basic assessment process

The objective of the basic assessment process is to, through a consultative process─

(a)  determine the policy and legislative context within which the proposed activity is located and how the activity complies with and responds to the policy and legislative context;

(b)  identify  the  alternatives  considered,  including  the  activity,  location,  and  technology alternatives;

(c)  describe the need and desirability of the proposed alternatives,

(d)  through the undertaking of an impact and risk assessment process inclusive of cumulative impacts  which focused on determining the geographical, physical, biological, social, economic, heritage , and cultural sensitivity of the sites and locations within sites and the risk of impact of the proposed activity and technology alternatives on the these aspects to determine:

(i)  the nature, significance, consequence, extent, duration, and probability of the impacts occurring to; and

(ii)  the degree to which these impacts—

(aa)  can be reversed;

(bb)  may cause irreplaceable loss of resources; and

(cc)  can be managed, avoided or mitigated;

(e)  through a ranking of the site sensitivities and possible impacts the activity and technology alternatives will impose on the sites and location identified through the life of the activity to—

(i)  identify and motivate a preferred site, activity and technology alternative;

(ii)  identify suitable measures to manage, avoid or mitigate identified impacts; and (iii)   identify residual risks that need to be managed and monitored.

PART A

SCOPE OF ASSSSMENT AND BASIC ASSESSMENT REPORT

- Contact Person and correspondence address

- Details of

  - Details of the EAP

Name of the EAP:  Mr. Daan Erasmus

Tel No.:  018 468 5355

Fax No. :  018 468 4015

E-mail address: daane@dera.co.za

  - Expertise of the EAP.

    - The qualifications of the EAP

(with evidence).

The EAP Mr. Daan Erasmus has a National Diploma in Agriculture Resource Utilization and a Baccalaureus Technologiae degree in Agricultural Extension.

See Figure 1 – Copy of qualification.

Figure 1 – Copy of Qualification

    - Summary of the EAP’s past experience	.

(In carrying out the Environmental Impact Assessment Procedure)

The EAP, Mr. Erasmus is involved in mining, environmental management, EMP & EMPR as well as Basic Assessments as from 1995. The EAP was involved in the NEMA Act through applications for chicken broilers where the Basic Assessments Report was also used to get to a ROD.

See Figure 2 - below Curriculum Vitae of D. E. Erasmus.

- Location of the overall Activity

Table 1: Property Description

| Farm Name: | Cyferfontein 333 IQ - (certain area of Portion 61 (portion of portion 39) |
| --- | --- |
| Application area (Ha) | 5 hectares |
| Magisterial district: | Vereeniging District |
| Distance and direction from nearest town: | Approximately 31 km north of Vereeninging and 13 km east of Ennerdale |
| 21 digit Surveyor General Code for each farm portion | T0IQ00000000033300061 |

- Locality map

(Show nearest town, scale not smaller than 1:250000).

See Appendix 1(a) attached for Locality Map.

- Description of the scope of the proposed overall activity.

Provide a plan drawn to a scale acceptable to the competent authority but not less than 1: 10 000 that shows the location, and area (hectares) of all the aforesaid main and listed activities, and infrastructure to be placed on site

Appendix 1(b) – Infrastructure/Site Plan.

- Listed and specified activities

Table 2: Listed Activities

| NAME OF ACTIVITY (E.g. For prospecting -drill site, site camp, ablution facility, accommodation, equipment storage, sample storage, site office, access route etc…etc…etc E.g. for mining,- excavations, blasting, stockpiles, discard dumps or dams, Loading, hauling and transport, Water supply dams and boreholes, accommodation, offices, ablution, stores, workshops, processing plant, storm water control, berms, roads, pipelines, power lines, conveyors, etc…etc…etc.) | Aerial extent of the Activity Ha or m² | LISTED ACTIVITY Mark with an “X” where applicable or affected. | APPLICABLE LISTING NOTICE (GNR 544, GNR 545 or GNR 546) | WASTE MANAGEMENT AUTHORISATION (Indicate whether authorisation is required in terms of the Waste Management Act). (Mark with an X) |
| --- | --- | --- | --- | --- |
| Mining Permit – excavations and processing (activity 21, listing 1) | 5 ha | X | GNR 327 |  |
| Clearing of an area more than 1 hectare (activity 27 listing 1) | 5 ha |  | GNR 327 |  |

- Description of the activities to be undertaken

(Describe Methodology or technology to be employed, including the type of commodity to be prospected/mined and for a linear activity, a description of the route of the activi

Table 3: Description of Activities to be followed

| ITEM | DESCRIPTION |
| --- | --- |
| Environmental attributes. Describe how the Environmental attributes associated with the development footprint will be determined. | The site was visited and a proper foot survey will be conducted. The activities that will be conducted by the applicant will be discussed on site as described in the Finance and technical document. The environmental setting on site and surrounding with the experience of the EAP will give an idea and lead to environmental attributes. |
| Identification of impacts and risks. Describe the process that will be used to identify impacts and risks. | The activities will take place will be discussed in detail with the applicant on site. With the specific environmental setting in mind and more specifically, the type of soil, soil depth, land use, vegetation type, and distances to open water and structures, the EAP will be able to identify potential impact areas where significant impacts might occur and the risks thereof. The methods of rehabilitation that need to be done, in order to meet the objective of the final land use will also be taken in consideration. |
| Consideration of alternatives. Describe how alternatives, and in particular the alternatives to the proposed site layout and possible alternative methods or technology to be applied will be determined. | The site will be visited before the BAR is compiled. There are no different site alternatives as this is a Mining Permit within the application area. The entire application area will be visited and areas that might be environmentally sensitive will be identified. The proposed impacts and mitigations will also be discussed. |
| Process to assess and rank impacts. Describe the process to be undertaken to identify, assess and rank the impacts and risks each individual activity, | The site was visited before the BAR was compiled. The entire application area will be visited and areas that might be environmentally sensitive will be identified. The proposed impacts and mitigations will also be discussed. The EAP (with 21 years’ experience in mining and mining activities) will assess the specific site for possible impacts. The assessment of impacts will be done according to a synthesis of the following assessment criteria: - Nature of the impact; - Extent (spatial scale); - Duration; - Magnitude or intensity of the impact (severity); - Probability; The criteria that will be used to determine significance as described below. Nature of the impact: This is an appraisal of the type of effect the activity would have on the affected environment. The description includes how and what is being affected, whether it is positive or negative, as well as whether it is direct or indirect. |
| Contribution of specialist reports. Describe how specialist reports, if required, will be taken into consideration and inform the impact identification, assessment and remediation process. | No specialist reports required at this stage, unless specifically requested. A heritage specialist will be involved with the BAR. |
| Determination of impact management objectives and outcomes. Describe how impact management objectives will be determined for each activity to address the potential impact at source, and how the impact management outcomes will be aligned with standards. | The Nature of the impact: This is an appraisal of the type of effect the activity would have on the affected environment. The description includes how and what is being affected, whether it is positive or negative, as well as whether it is direct or indirect. Each impact will be assessed and quantified, and management objectives according to the first two steps, will be set. The management of the objectives will be aligned with the significance of the impact, as well as to ensure a positive outcome. The outcomes will be aligned with standards on environmental management and rehabilitation of mining areas according to Department Mineral Resources. |

- The mineral

Raubex Construction (Pty) Ltd. intends to mine for Gravel (grav) G5 stone; Aggregate gravel (st) situated on a portion of the farm Cyferfontein 333 IQ, Vereeniging district, and 5 hectares in total.  The gravel/stone will be used as filling material in road construction. The gravel will be used in different facets of the road construction industry.

- The extend

The gravel is situated on this demarcated area on average 4 meters deep.  The identified and demarcated which are 5 hectares in total includes the entire mining area of 3.5 ha will be used for mining and 1.5 ha for the stockpiling and plant area.  The gravel reserve on this 3.5 hectares is estimated at 140 000 m³.

- Mining method

No processing only load and haul. The above area will be mined through opencast excavations where the gravel will be removed with an excavator onto a stockpile and the gravel will be directly loaded onto the trucks. A stockpile will be created and loaded on the trucks for transporting to the clients. It is envisaged that most of the gravel will be loaded directly onto the trucks without any processing. The gravel from the stockpile is transported at an average rate of 450 m a day to the clients or as needed.

The total estimated reserve of gravel is 140 000 m³ taken at a production rate of 9000 m³ a month it will take 16 months to work this reserve.

The gravel which is 4 m thick and the relatively low production rate of this operation make this 5 hectare to be worked sustainable over a period of two years.

- Equipment to be used include:

- 2 x Frond end loaders;

- 1 x Excavator;

- 4 x trucks;

- 1 x screening/crushing plant.

- 10 x Permanent labourers and one manager will used in this operation.

The total cost of the operation is taken at R 34/ton and the total material moved monthly at 9000 m³. The total monthly mining cost is then R 314 199 .00 and the total monthly income is on average R 495 801.00. This operation can thus be economical viable.

The grade

It is estimated that this sand will be sold for R 90.00/m³.

- Policy and Legislative Context

Table 4: Policy & Legislative Context

| APPLICABLE LEGISLATION AND GUIDELINES USED TO COMPILE THE REPORT (a description of the policy and legislative context within which the development is proposed including an identification of all legislation, policies, plans, guidelines, spatial tools, municipal development planning frameworks and instruments that are applicable to this activity and are to be considered in the assessment process | REFERENCE WHERE APPLIED | HOW DOES THIS DEVELOPMENT COMPLIY WITH AND RESPOND TO THE LEGISLATION AND POLICY CONTEXT. (E.g. In terms of the National Water Act a Water Use License has/ has not been applied for) |
| --- | --- | --- |
| Mineral & Petroleum Resources Development Act 28 of 2002 |  | Submitted for Environmental Authorizations in terms of the National Environmental Management Act, 1998 and the National Environmental Management Waste Act, 2008 in respect of Listed Activities that has been triggered by applications in terms of the Minerals and Petroleum Resources Development Act, 2002 (As mentioned). |
| National Environmental Management Act, 1998 (Act 107 of 1998) (NEMA) |  | EA Authorization and BAR |
| National Environmental Management Act, 1998 (Act 107 of 1998): Environmental Impact Assessment Regulations, 2014 (G38282 – R982-985) |  | Compliance to Act and Regulations during course of activities. |
| World Heritages Convention Act, 1999 (Act 49 of 1999) |  | Compliance to Act and Regulations during course of activities. |
| Environmental Conservation Act, 1989 (Act 73 of 1989) |  | Compliance to Act and Regulations during course of activities. |
| Conservation of Agricultural Resources Act, No. 43 of 1983 |  | Compliance to Act and Regulations during course of activities. |

- Need and desirability of the proposed activities

(Motivate the need and desirability of the proposed development including the need and desirability of the activity in the context of the preferred location).

This will be a very small project consists of only excavation, stockpile and then the gravel will be transported to be used in road construction. The portions over which the application was applied for is over an area that seems to be natural veld.  There don’t seem to be any pre-mining disturbance and the area is under natural vegetation.   The structures found on site are an entrance road. Access to the farm is gained by an existing gravel road from the R 557 tar road.  See Figure 3 below for Google Earth Images of proposed area.  The area will be mined and rehabilitated. The mining focus area will be clearly demarcated. The area applied for is over the demarcated portion only.  After mining the land will be rehabilitated and could be used for grazing again. See photos attached as Appendix 1c of the application area.

Figure 3: Google Earth Images

- Motivation for the overall preferred site, activities and technology alternative

The applicant envisaged that the applied mineral (Gravel (grav) G5 stone; Aggregate gravel (st)) might be present on this property and therefore the application for a mining permit. The gravel does outcrop on several places within the application area and for this reason this area is the preferred area.

- Full  description  of  the  process  followed  to  reach  the  proposed  preferred alternatives within the site

NB!! – This section  is about the determination of the specific site layout and the location of infrastructure and activities on site, having taken into consideration the issues raised by interested and affected parties, and the consideration of alternatives to the initially proposed site layout.

- Details of the development footprint alternatives considered

With reference to the site plan provided as Appendix 4 and the location of the individual activities on site, provide details of the alternatives considered with respect to:

Alternative is not applicable.  The current land is natural veld. Thus the option to mine the area will be an alternative land use.  Mining the area and rehabilitation the area will ensure that the area could be used for agricultural practices again.  The applicant, Raubex Construction (Pty) Ltd, is not interested in any other alternative land use over this land aside for the exploration of Gravel (grav) G5 stone and Aggregate gravel (st), or any other activity, or method use other than mining for G5 gravel in the conventional way, which is the most cost effective.

(a)	  the property on which or location where it is proposed to undertake the activity

There are no alternative for the property as the application is for this 5 hectare area only.

(b)  the type of activity to be undertaken

The type of activity is in line with the submitted Mining Plan.

(c) the design or layout of the activity

The layout of the activity will and can only be on the application area as per sketch plan.

(d) the technology to be used in the activity

The technology used in the activity will as described in the Mining Plan and the best options will be determined by the applicant.

(e) the operational aspects of the activity, and

The operational aspect is only the mining of gravel on this specific area.

(f) the option of not implementing the activity

This option might only be possible if the applicant decide to abandon the project.

- Details of the Public Participation Process Followed

Describe the process undertaken to consult interested and affected parties including public meetings and one on one consultation. NB the affected parties must be specifically consulted regardless of whether or not they attended public meetings. (Information to be provided to affected parties must include sufficient detail of the intended operation to enable them to assess what impact the activities will have on them or on the use of their land.

The process as described by NEMA for Environmental Authorization was followed. See Table 5 & 6 below for the identification of Interested and Affected Parties to be consulted with. The landowner (Zion Christian Church Zimbabwe), occupiers and the direct neighbours were consulted personally and through a letter that was given to them by hand.  A site notice was placed at the entrance gate. With this site notice all passersby are requested to submit any written comments to be forwarded to the consultant. An advertisement was  placed in the local newspaper, the Vaalweekblad of 21 March 2018. See proof of consultation under Appendix 2.

Appendix 2 – Proof of consultation.

- Details of the Public Participation Process Followed

Describe the process undertaken to consult interested and affected parties including public meetings and one on one consultation. NB the affected parties must be specifically consulted regardless of whether or not they attended public meetings. (Information to be provided to affected parties must include sufficient detail of the intended operation to enable them to assess what impact the activities will have on them or on the use of their land.

Table 5: Identification of Interested and Affected Parties to be consulted

| IDENTIFICATION CRITERIA | Mark with an X where applicable |  |
| --- | --- | --- |
|  | YES | NO |
| Will the landowner be specifically consulted? | X |  |
| Will the lawful occupier on the property other than the Landowner be consulted? | X |  |
| Will a tribal authority or host community that may be affected be consulted? |  | X |
| Will recipients of land claims in respect of the area be consulted? | X |  |
| Will the landowners or lawful occupiers of neighbouring properties been identified? | X |  |
| Will the local municipality be consulted? | X |  |
| Will the Authority responsible for power lines within 100 metres of the area be consulted? |  | X |
| Will the Authorities responsible for public roads or railway lines within 100 metres of the area applied for be consulted? |  | X |
| Will the Authorities responsible for any other infrastructure within 100 metres the area applied for be consulted? (Specify) |  | X |
| Will the Provincial Department responsible for the environment be consulted? | X |  |
| Will all of the parties identified above be provided with a description of the proposed mining/prospecting operation as referred above? | X |  |
| Will all the parties identified above be requested in writing to provide information as to how their interests (whether it be socio-economic, cultural, heritage or environmental) will be affected by the proposed mining project? | X |  |
| Other, Specify |  |  |

Table 6: Furthermore the details of the engagement process to be followed are as reflected below.

| Steps to be taken to notify interested and affected parties (Describe the process to be undertaken to consult interested and affected parties including public meetings and one on one consultation. NB the affected parties must be specifically consulted regardless of whether or not they attended public meetings. Photographs of notice and copies of advertisements and notices notifying potentially interested and affected parties of the proposed application are attached as Appendix 1(b). | PROVIDE DESCRIPTION HERE The applicant does have a surface lease agreement with the landowner and the neighbours was informed personally consulted by the applicant and confirmed in the writing. A consultation letter was send to the Local Municipality. An advertisement was placed in the local newspaper for comments. |
| --- | --- |
| Information to be provided to Interested and Affected Parties. | Compulsory - The site plan. - List of activities to be authorized - Scale and extent of activities to be authorized - Typical impacts of activities to be authorized (e.g. surface disturbance, dust, noise, drainage, fly rock etc.) - The duration of the activity. - Sufficient detail of the intended operation to enable them to assess what impact the activities will have on them or on the use of their land) |
|  | Other, specify: mining plan |
| Information to be required from Interested and Affected Parties. | Compulsory - To provide information on how they consider that the proposed activities will impact on them or their socio-economic conditions - To provide written responses stating their suggestions to mitigate the anticipated impacts of each activity - To provide information on current land uses and their location within the area under consideration - To provide information on the location of environmental features on site to make proposals as to how and to what standard the impacts on site can be remedied. requested to make written proposals - To mitigate the potential impacts on their socio economic conditions to make proposals as to how the potential impacts on their infrastructure can be managed, avoided or remedied). |
|  | Other, Specify |

- Summary of issues raised by I&AP’s

(Complete the table summarising comments and issues raised, and reaction to those responses)

See Appendix 2 for full detail on public participation.

Table 7: Summary of Identified I&AP’s

| Interested and Affected Parties List the names of persons consulted in this column, and Mark with an “X” where those who must be consulted were in fact consulted. |  | Date sent and/or Comments Received | Issues raised | EAP’s response to the applicant |
| --- | --- | --- | --- | --- |
| AFFECTED PARTIES |  |  |  |  |
| Zion Christian Church Zimbabwe (Landowner) | X | 16 Jan 2018 20 March 2018 | Consultation still in process Applicant still trying to get hold of the landowner to consult with |  |
| Lawful occupier/s of the land |  |  |  |  |
| Landowners or lawful occupiers on adjacent properties | X |  |  |  |
| (Neighbours) |  | 16 Jan 2018 20 March 2018 | Consultation still in process |  |
| Municipal councilor |  |  |  |  |
| Municipality | X |  |  |  |
| Emfuleni Local Municipality LED officer: Sibusiso Biyela Tel: 016 950 5000 Fax: 016 950 5050 |  | 16 Jan 2018 | No response |  |
| Organs of state (Responsible for infrastructure that may be affected Roads Department, Eskom, Telkom, DWA. |  |  |  |  |
| Eskom |  |  |  |  |
| Communities |  |  |  |  |
| Dept. Land Affairs | X |  |  |  |
| Cindy Benyane cindy.benyane@drdlr.gov.za |  | 16 Jan 2018 1 March 2018 | Awaiting response Comments received 1 March 2018 – Existing land claim |  |
| Traditional Leaders |  |  |  |  |
| N/A |  |  |  |  |
| Dept. Rural, Environment and Agricultural Development | X |  |  |  |
| Mr Andile A. Gumede Umnotho House, 56 Eloff & Fox Street, Johannesburg, 2000 Tel: 011 355 1957 E-mail: andile.gumede@gauteng.gov.za; Dec 2016 25 May 2017 13 June 2017; Scoping Report was sent via e-mail – No response. EMPr/EIA was sent with Fastway couriers for comments. Acknowledgement letter received – no comments received Awaiting response.. |  | 1 Febr 2018 | BAR was sent with Fastway couriers for comments | No comments received |
| Dept. Water and Sanitation | X |  |  |  |
| Mr. Sibusiso mthembu Ndinaye House, 178 Francis Baard Street, Pretoria, 0001 Tel: 012 336 8065 / 082 885 3915; Dec 2016 25 May 2017 13 June 2017; Scoping Report was sent via e-mail – NoNo response. EMPr/EIA was sent with Fastway couriers for comments. Acknowledgement letter received – no comments received Awaiting response.. |  | 1 Febr 2018 | BAR was sent with Fastway couriers for comments . Awaiting response. | No comments received |
| Dept. Agriculture, Forestry and Fisheries | X |  |  |  |
| Ms Phyllystas Mmakola Delpen Building, C/O Union street & Annie Botha Avenue, Riviera, Pretoria, 0001 Tel: 012 319 7484 / 082 404 3054 E-mail: PhyllystasM@daff.gov.za |  | 1 Febr 2018 | BAR was sent with Fastway couriers for comments . | No comments received |
| Dept. Rural Development and Landform | X |  |  |  |
| Cindy Benyane Tel: 012 310 6620 E-mail: Cindy.benyane@drdlr.gov.za |  | 1 Febr 2018 | BAR was sent by e-mail for comments | No comments received |
| Other Competent Authorities | X |  |  |  |
| OTHER Affected Parties |  |  |  |  |
| INTERESTED PARTIES |  |  |  |  |

Notice published in Vaalweekblad 21 March 2018

- The Environmental attributes associated with the alternatives.

(The environmental attributed described must include socio-economic, social, heritage, cultural, geographical, physical and biological aspects)

  - Baseline Environment

Introduction:

The purpose of this section is to provide information on the environment in which the proposed mining activities will take place, with a view to identify sensitive issues/areas, which need to be considered when conducting the impact assessment.

The application is over: a certain area of Portion 61 (portion of portion 39) of the farm Cyferfontein 333 IQ.  This area can be described as natural vegetation, see Figure 3 Google Earth Images.

Magisterial District:

Midvaal Local Municipality, which form part of Sedibeng District Municipality.

Direction from neighbouring town:

The site is situated approximately 12 min (10.2 km) via R558, SAPS – Ennerdale, 1830.  Or 36 min (31.0 km) via R82, SAPS – Vereeniging, 40 Voortrekker Street, Vereeniging, 1939.  From  SAPS - Ennerdale head southeast on Louw Cres toward Town Road for 280 m.  Turn right at the 1st cross street onto Town Road continue for 900 m.  Turn left onto R558 drive for 5.9 km.  Turn left onto R550/R558 drive for 600 m.  Turn right at the 1st cross street onto R557 drive for 2.3 km.  The proposed site will be on the left in 160 m.  See Location of proposed site on Locality Map Appendix 1(b).

Longitude (approximate center of mining site):

27.9129 E

Latitude (approximate centre of mining site):

-26.4264 S

Existing Surface Infrastructure:

The existing infrastructure consists of an entrance road and small holding infrastructure.  See Appendix 1(a) for Infrastructure Map.

(a)  Type of environment affected by the proposed activity.

(its current geographical, physical, biological, socio- economic, and cultural character).

According VEGMAP (2006) classified this area as part of the Gm 8 Soweto Highveld Grassland area.  VT 52 Themeda Veld (Turf Highveld) (56%) (Acocks 1953). LR 35 Moist Clay Highveld Grassland (51%) (Low & Rebelo 1996).

Distribution: Mpumalanga, Gauteng (and to a very small extent also in neighbouring Free State and North-West) Provinces: In a broad band roughly delimited by the N17 road between Ermelo and Johannesburg in the north, Perdekop in the southeast and the Vaal River (border with the Free State) in the south.  It extends further westwards along the southern edge of the Johannesburg Dome (including part of Soweto) as far as the vicinity of Randfontein. In southern Gauteng it includes the surrounds of Vanderbijlpark and Vereeniging as well as Sasolburg in the northern Free State. Altitude 1 420-1 760 m..

Climate:

Summer-rainfall region (MAP 662 mm). Cool-temperate climate with thermic continentality (high extremes between maximum summer and minimum winter temperatures, frequent occurrence of frost, large thermic diurnal differences, especially in autumn and spring).

Geology and Soils:

Geology and Soils Shale, sandstone or mudstone of the Madzaringwe Formation (Karoo Supergroup) or the intrusive Karoo Suite dolerites which feature prominently in the area. In the south, the Volksrust Formation (Karoo Supergroup) is found and in the west, the rocks of the older Transvaal, Ventersdorp and Witwatersrand Supergroups are most significant. Soils are deep, reddish on flat plains and are typically Ea, Ba and Bb land types.

Vegetation [Flora] and Landscape Features:

Gently to moderately undulating landscape on the Highveld plateau supporting short to medium-high, dense, tufted grassland dominated almost entirely by Themeda triandra and accompanied by a variety other grasses such as Elionurus muticus, Eragrostis racemosa, Heteropogon contortus and Tristachya leucothrix. In places not disturbed, only scattered small wetlands, narrow stream alluvia, pans and occasional ridges or rocky outcrops interrupt the continuous grassland cover.

Important Taxa:

Graminoids: Andropogon appendiculatus Brachiaria serrata (d), Cymbopogon pospischilii (d), Cynodon

dactylon (d), Elionurus muticus (d), Eragrostis capensis (d), E. chloromelas (d), E. curvula (d), E. plana (d), E. planiculmis (d), E. racemosa (d), Heteropogon contortus (d), Hyparrhenia hirta (d), Setaria nigrirostris (d), S. sphacelata (d), Themeda triandra (d), Tristachya leucothrix (d), Andropogon schirensis, Aristida adscensionis,  A. bipartita, A. congesta, A. junciformis subsp. galpinii, Cymbopogon caesius, Digitaria diagonalis, Diheteropogon amplectens, Eragrostis micrantha, E. superba, Harpochloa falx, Microchloa caffra, Paspalum dilatatum. Herbs: Hermannia depressa (d), Acalypha angustata, Berkheya setifera, Dicoma anomala, Euryops gilfillanii, Geigeria aspera var. aspera, Graderia subintegra, Haplocarpha scaposa, Helichrysum miconiifolium, H. nudifolium var. nudifolium, H. rugulosum, Hibiscus pusillus, Justicia anagalloides, Lippia scaberrima, Rhynchosia effusa, Schistostephium crataegifolium, Selago densiflora, Senecio coronatus, Vemonia oligocephala, Wahlenbergia undulata. Geophytic Herbs: Haemanthus humilis subsp. hirsutus, H. montanus. Herbaceous Climber: Rhynchosia totta. Shrubs: Anthospermum hispidulum, A. rigidum subsp. pumilum, Berkheya annectens, Felicia muricata, Ziziphus zeyheriana.  See Figure 7 below.

Conservation:

Conservation Endangered. Target 24%. Only a handful of patches statutorily conserved (Waldrift, Krugersdorp, Leeuwkuil, Suikerbosrand, Rolfe's Pan Nature Reserves) or privately con-Tweefontein, Gert Jacobs, Nikolaas and Avalon Nature Reserves, Heidelberg Natural Heritage Site). Almost half of the area already transformed by cultivation, urban sprawl, mining and build-ing of road infrastructure. Some areas have been flooded by dams (Grootdraai, Leeukuil, Trichardtsfontein, Vaal, Willem Brummer). Erosion is generally very low (93%).

Figure 4: (Gm 8) Soweto Highveld Grassland

Animal Life [Fauna]:

Small animals common in this area include: Steenbuck, Duiker, Jackal and Meerkat and Guinea Fowl.

Topography:

Gently to moderately undulating landscape on the Highveld plateau. The slope varies around <0.2% to not more than 2%.

Surface Water:

This site falls in Middle Vaal (8) water management area as classified by the Department of Water Affairs, under tertiary drainage region C22 and quaternary catchment C22E.  There is no open water or streams within 5km distance of the application area. River diversion is not applicable.

Ground Water:

Presence of water boreholes and springs: There are no boreholes on the application area.

Ground water use: The applicant intends to use water from current boreholes at the municipal land. The water uses will be 2m³ a day for the dust suppression only. The current ground water uses in the vicinity of the study area are mining and residential.

Air Quality:

The air quality of the pre-mining period is expected to have been of a better quality than current. However, mining activities in the surrounding areas also contribute to the air quality degradation. The impact on air quality will only start with the bulk sampling, where dust from excavating and from the roads will occur. This impact will be low and will be monitored and mitigated through suppressing the dust by wetting the roads.

Noise:

The impact of noise will only start where noise from the mining equipment will be generated. This operation will only be in day time working hours and will have a low impact on current surroundings.

Sites of Archaeological and Cultural Interest:

No graves or other archaeological/cultural sites were identified on the application area. However, should any archaeological sites be discovered, all work will be ceased and the relevant specialist will be contacted in conjunction with SAHRA, and the appropriate steps will be taken to protect the identified resource.

According to Section 36(3) of the National Heritage Resources Act 25 of 1999 no person may, without a permit issued by SAHRA or a provincial heritage resources authority—

(a)	destroy, damage, alter, exhume or remove from its original position or otherwise disturb the grave of a victim of conflict, or any burial ground or part thereof which contains such graves;

(b) destroy, damage, alter, exhume, remove from its original position or otherwise disturb any grave or burial ground older than 60 years which is situated outside a formal cemetery administered by a local authority; or

(b)	bring onto or use at a burial ground or grave referred to in paragraph (a) or (b) any excavation equipment, or any equipment which assists in the detection or recovery of metals.

It is recommended that the graveyard is included in the overall management plan of the mine development. Preservation of the site will require that the area is properly demarcated with at least a 20m buffer zone placed around the graveyard in order to avoid potential damage during mining activities. It will be necessary to ensure that the graveyard is accessible to the relatives of the deceased.

There are no major archaeological grounds to halt the proposed development. However, the potential occurrence of unmarked graves or subsurface finds not recorded during this survey can never be excluded, so it is advised that SAHRA and a qualified archaeologist are informed immediately if archaeological objects are uncovered.

Sensitive Landscapes

There were no sensitive landscapes identified on the site visit.

Visual Aspects

These mining activities will be partly visible from the R557 and to passersby and adjacent neighbor. There are no mitigated as the site is situated next to the road.

Social

The proposed activity will employ 6 people, of which a few are resident around the operation. Various social amenities are available close to the operation. These include schools, hospitals churches, recreation facilities as well as a Police Station at Ennerdale, which is located approximately 10 km west of the operation.

- Description of the current land uses.

The current land use natural vegetation. The surrounding areas are used for different activities like farming.

- Description of specific environmental features and infrastructure on the site.

Please refer to Section 2 (d)(ii) Table 2 for a description of the activities and the infrastructure which are foreseen to form part of the mining activity.  The existing infrastructure on site consists only out natural vegetation. See Figure 3 of existing infrastructure.

- Environmental and current land use map.

(Show all environmental and current land use features)

Current land use of the application area is natural vegetation. See Appendix 1a [Infrastructure Map] and Figure 3 [Images of existing infrastructure] for more detail.

- Impacts  and  risks  identified  including  the  nature,  significance, consequence, extent, duration and probability of the impacts, including the degree to which these impacts

(Provide a list of the potential impacts identified of the activities described in the initial site layout that will be undertaken, as informed by both the typical known impacts of such activities, and as informed by the consultations with affected parties together with the significance, probability, and duration of the impacts. Please indicate the extent to which they can be reversed, the extent to which they may cause irreplaceable loss of resources, and can be avoided, managed or mitigated).

The proposed project is anticipated to impact on a range of biophysical and socio-economic aspects of the environment. The main purpose of the Basic Assessment Report is to identify and evaluate the significance of these potential impacts and determine how they can be minimized or mitigated.

It should be noted that a comprehensive Environmental Management Program (EMPr) will be developed and implemented to regulate and minimize the direct, indirect and cumulative impacts during the construction and operational phases. The potential environmental impacts identified, which will be investigated further in the Impact Assessment Phase of the project, are summarized in Table 8 on next page.

Table 9: Impact significance identification matrix for Cyferfontein 333 IQ

|  |  |  | A | B | C | D | E |  | F | E | F |  | G | H |  | I | J | K | L | M | N |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PHASE | Components | ABIOTIC |  |  |  |  |  |  |  |  |  |  | BIOTIC |  |  |  | VISUAL | SOCIO-ECONOMIC |  |  |
|  |  | Impacts | Geology | Topography | Soil | Land capability | Land use potential | Surface water |  | Ground water | Air quality | Noise |  | Vegetation | Wildlife |  | Sensitive landscapes | Visual impact | Archaeological & cultural sites | Socio-economic impacts | Affected parties |
|  |  | Activity, Product or Service |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Construction | Demarcation of mine focuses area. |  |  | L | M | L |  |  |  |  |  |  |  | M |  |  | M |  |  |  |
| 2 |  | Establishment (site preparation, vegetation clearance, topsoil removal and stockpiling) of proper access roads (upgrade existing road), Initial vegetation clearance, topsoil removal & stockpiling next to first opencast/pit/trench within the mine focus area. |  | M | H | H |  |  |  | M | H | H |  | H | H |  | L | M |  | L | M |
| 3 |  | Establishment of bunded diesel and oil/chemical storage facilities, chemical toilets. |  | M | M | H |  | M |  | M |  |  |  | M |  |  |  | M |  |  |  |
| 4 |  | Provision of storage tanks for potable (drinking water) and process water (dust suppression). |  | H | H | H | H | L |  | M | M | H |  | H | H |  | M | L | L |  |  |
| 5 |  | Provision of waste handling/disposal facilities (domestic & industrial waste bins. |  |  | L |  |  | L |  | L |  |  |  |  |  |  | L |  |  |  |  |
| 6 |  | Fencing –off active mining site in as required in terms of the MHSA. Ensure access control (gate), ect. |  |  |  | M |  |  |  |  |  |  |  |  | M |  |  | M |  |  | H+ |
| 7 | Operational | Vegetation clearance, topsoil removal & stockpiling net to opencast/pit/trench within the mine focus area (0.5 ha of surface area disturbed at any given time). |  | M | H | H | M | L |  |  | L |  |  | H | L |  |  | L |  | M | H |
| 8 |  | Mechanically excavating overburden with an excavator and stockpile separately from topsoil dump. Remove sand with excavator and stockpile on side of /pit to load onto trucks. | H | M+ | H | H | H | L |  | M | L | L |  | H | L |  |  | L+ |  | M | H |
| 9 |  | Transport of sand away from the site. |  |  | H |  |  | L |  | H | L | L |  | H |  |  |  | M |  | M | H |
| 11 |  | Final sloping of all voids/trenches. | H+ | H+ | H+ | H+ | H+ | H+ |  | H+ | L | L |  |  |  |  |  | L |  | H+ | H± |
| 13 | Decommissioning and closure | Replace and spread all topsoil evenly over sloped sites. |  |  | H+ | H+ | H+ | H+ |  | H+ | H+ | L |  | H+ | H+ |  |  | H+ |  | H+ | H+ |
| 14 |  | Establishment of vegetation cover. |  |  | H+ | H+ | H+ | H+ |  | H+ | H+ |  |  | H+ | H+ |  |  | H+ |  | H+ | H+ |
| 15 |  | Removal of all temporary & demolition of all permanent structures (Section 44 of the MPRDA). |  |  | H+ | H+ | H+ | H+ |  | H+ | H+ | L |  | H+ | H+ |  |  | H+ |  | H+ | H+ |
| 16 |  | Rehabilitation of all access roads, compacted areas, etc. |  |  | H+ | H+ | H+ | H+ |  | H+ | H+ | L |  | H+ | H+ |  |  | H+ |  | H+ | H+ |

- Methodology used in determining and ranking the nature, significance, consequences,  extent,  duration  and  probability  of  potential environmental impacts and risks;

(Describe how the significance, probability, and duration of the aforesaid identified impacts that were identified through the consultation process was determined in order to decide the extent to which the initial site layout needs revision).

Introduction:

This section below describes and evaluates the effects of the different mining projects and the associated activities on the natural and social environments. The different environmental components, on which the project (can/may) have an impact, are:

1.	Geology

2.	Topography

3.	Soil

4.	Land Capability

5.	Land Use

6.	Vegetation

7.	Wildlife

8.	Surface Water

9.	Ground Water

10.	Air Quality

11.	Noise

12.	Archaeological and Cultural sites

13.	Sensitive Landscapes

14.	Visual Aspects

15.	Socio-economic Structure

16.	Interested and Affected Parties

IMPACT ASSESSMENT

Before the impact assessment could be done the different project activities were identified:

ACTIVITIES:

- Access Roads (Existing farm roads to be upgraded)

- Temporary office, workshops, ablution facility, water tanks, diesel tanks  and other temporary buildings

- Mining equipment ( conveyor, screen/crusher, generator)

- Stockpiles

- Opencast trenches (as part of bulk sampling)

## Environmental Impact Assessment Summary:

- Environment likely to be affected by the mining operation.  (See Appendix 1 (b) for location)

| Environmental aspect | Affected |  | Not affected |
| --- | --- | --- | --- |
|  | Negligible | Substantial |  |
| 1. GEOLOGY |  | X |  |
| 2. TOPOGRAPHY | X |  |  |
| 3. SOIL |  | X |  |
| 4. LAND CAPABILITY |  | X |  |
| 5. LAND USE | X |  |  |
| 6. VEGETATION |  | X |  |
| 7. WILDLIFE | X |  |  |
| 8. SURFACE WATER |  |  | X |
| 9. GROUND WATER | X |  |  |
| 10. AIR QUALITY | X |  |  |
| 11. NOISE | X |  |  |
| 12. SENSITIVE LANDSCAPES |  |  | X |
| 13. VISUAL ASPECTS | X |  |  |
| 14. SOCIO ECONOMICS | X |  |  |
| 15. INTERESTED & AFFECTED PARTIES | X |  |  |
| 16. ARCHAEOLOGICAL |  |  | X |

- Environment likely to be affected by the alternative land use

Mining will not be a new land use over this area.  The site that is earmarked for mining represents ± 100 % of the total area applied for.  And it is further not foreseen that mining activities would disturbed an area of more than 0.5 ha at any given time.  The rest of the terrain would continue to be used for agriculture purposes by the landowner.

- Assessment of the impacts created by the mining activity

Before any assessment can be made the following evaluation criteria need to be described:

Explanation of probability of impact occurrence

| Probability of impact occurrence | Explanation of probability |
| --- | --- |
| Very low | <20% sure of particular fact or likelihood of impact occurring. |
| Low | 20 to 39% sure of particular fact or likelihood of impact occurring. |
| Moderate | 40 to 59% sure of particular fact or likelihood of impact occurring. |
| High | 60 to 79% sure of particular fact or likelihood of impact occurring. |
| Very high | 80 to 99% sure of particular fact or likelihood of impact occurring. |
| Definite | 100% sure of particular fact or likelihood of impact occurring. |

Explanation of extend of impact

| Extend of impact | Explanation of extend |
| --- | --- |
| Site specific | Direct and indirect impacts limited to site of impact only. |
| Local | Direct and indirect impacts affecting environmental elements within the Midvaal Local Municipality. |
| Regional | Direct and indirect impacts affecting environmental elements within Gauteng Province. |
| National | Direct and indirect impacts affecting environmental elements on a national level. |
| Global | Direct and indirect impacts affecting environmental elements on a global level. |

Explanation of duration of impact

| Duration of impact | Explanation of duration |
| --- | --- |
| Very short | Less than 1 year |
| Short | 1 to 5 years |
| Medium | 6 to 12 years |
| Long | 13 to 50 years |
| Very long | Longer than 50 years |
| Permanent | Permanent |

Explanation of impact significance

| Impact significance | Explanation of significance |
| --- | --- |
| No impact | There would be no impact at all - not even a very low impact on the system or any of its parts. |
| Very low | Impact would be negligible. In the case of negative impacts, almost no mitigation and/or remedial activity would be needed, and any minor steps, which might be needed, would be easy, cheap and simple. In the case of positive impacts, alternative means would almost all likely to be better, in one or a number of ways, than this means of achieving the benefit. |
| Low | Impact would be of a low order and with little real effect. In the case of negative impacts, mitigation and/or remedial activity would be either easily achieved or little would be required, or both. In case of positive impacts, alternative means for achieving this benefit would likely be easier, cheaper, more effective, less time-consuming, or some combination of these. |
| Moderate significance | Impact would be real but not substantial within the bounds of those which could occur. In the case of negative impacts, mitigation and/or remedial activity would be both feasible and fairly easily possible. In the case of positive impacts, other means of achieving these benefits would be about equal in time, cost and effort. |
| High significance | Impacts of a substantial order. In the case of negative impacts, mitigation and/or remedial activity would be feasible but difficult, expensive, time-consuming or some combination of these. In the case of positive impacts, other means of achieving this benefit would be feasible, but these would be more difficult, expensive, time-consuming or some combination of these. |
| Very high significance | Of the highest order possible within the bounds of impacts which could occur. In the case of negative impacts, there would be no possible mitigation and/or remedial activity to offset the impact at the spatial or time scale for which it was predicted. In the case of positive impacts, there is no real alternative to achieving the benefit. |

- The positive and negative impacts that the proposed activity (in terms of the initial site layout) and alternatives will have on the environment and the community that may be affected.

(Provide a discussion in terms of advantages and disadvantages of the initial site layout compared to alternative layout options to accommodate concerns raised by affected parties)

In terms of the EIA regulations, consideration must be given to alternatives. Alternatives are different approaches and ways of meeting the need, purpose and objectives of a proposed activity. Alternatives may include a location site alternative, activity alternatives, processes or technology alternatives, temporal alternatives etc. the no-go alternative or option is also considered, as it provides the baseline against which the impacts or other alternatives may be compared.

However, for this specific project, no alternatives have been investigated, with the exception of the no-go alternative. The reason for this being that the mining permit is being applied for the sole purpose of mining of gravel. The no-go option entails the continuation of the current land use (natural grazing) on the study site. The project will contribute towards providing continued jobs for current staff. Should the proposed project therefore not be authorized to proceed, it is anticipated that current employment opportunities will be terminated once the mineral reserves have been depleted.

The no-go option is therefore not a feasible option in this case, as it suggests that the mineral reserves should not be exploited and current employment opportunities should not materialize or be prolonged.

The site layout will be only the excavation and the plant and office container. The stockpiles of the topsoil will be placed next to the side walls of the excavation on the outside. This will have the advantage to be nearby available to be used for rehabilitation. The stockpiles for the gravel (product) and the screening/crushing plant will be placed just outside the excavation within the mining area which will have the advantage that the loading of trucks can proceed without hampering the mining process and will be a safer mining environment.

- The possible mitigation measures that could be applied and the level of risk.

(With regard to the issues and concerns raised by affected parties provide a list of the issues raised and an assessment/ discussion of the mitigations or site layout alternatives available to accommodate or address their concerns, together with an assessment of the impacts or risks associated with the mitigation or alternatives considered).

Refer to the results of consultation contained in Table 7 for the issues that were raised by I&AP’s and stakeholders during the review period of the Consultation phase of the BAR/EMPr report, as well as the response to those issues made by the Environmental Assessment Practitioner.

The mitigation measures and technical management action plans which address potential impacts are discussed below. Please see section below for more detail.

Table 10: Assessment of the nature, extent, duration, probability and significance of the potential environmental, social and cultural impacts of the proposed mining operation, including the cumulative environmental impacts.

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 1. GEOLOGY |  |  |  |  |  |
| Nature of the impact | Geology (deposits will be destroyed during the opencast mining operation. During operation which will be for the next 2 years, the mineral resource (G5 Gravel) will be extracted from mineral deposits. Waste rock material/overburden material is disposed off/backfilled in existing excavations as part of the mining process. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Permanent |  |  |  | An opencast mining method will be used to extract mineral deposits. Therefore the original geology will be totally destroyed. |
| Probability | Definite |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 2. TOPOGRAPHY |  |  |  |  |  |
| Nature of the impact | * Change in landform : * The mining site is situated on: level plains some relief. * Disturbance of the surface drainage: The mining of the mineral deposits will result in the creation of excavations (20 x 10 m x 3-5 m or less), that act as depressions in the environment that captures run-off. Mining activities will be concentrated as indicated on Appendix 1(b) on the application area (approximately 3-5 m depth). Normal surface drainage will be disturbed at a given point. Run-off if any will be diverted away from the specific site. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Very long to Permanent |  |  |  | Creation of excavations |
| Probability | Definite |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| 3. SOIL | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| Nature of the impact | The surface area is characterized by various soil depths as the area was disturbed before. Any construction of infrastructure should be preceded by the removal of all available topsoil where available. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | In the process of removing topsoil the soil layers are mixed and the structure may be disturbed. |
| Probability | High |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| 3. SOIL | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| Nature of the impact | The establishment, construction, operation and eventually rehabilitation (demolition) of listed structures such as the access roads, stockpiles /tailings dumps, cause compaction of soil. All mining activities will be concentrated on the identified mining focus area where mineral deposits could be found. In the same time a certain surface area is therefore alienated. The active mining surface area (alienated) would be restricted within the ±0.5 ha at any given time (in relation to area of application of the mining permit of 5 ha) for the next 2 years. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Site preparation for additional mining sites and the construction, operation of listed infrastructure. |
| Probability | High |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 3. SOIL |  |  |  |  |  |
| Nature of the impact | Soil erosion: Due to the fact that certain surface areas would become compacted and this would lead to lesser infiltration of rainwater and more run-off that could cause erosion on bare disturbed surfaces. Erosion would always be possible until such time a vegetation cover is provided during rehabilitation phase. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Very short |  |  |  | When removing topsoil during site preparation, little storm water control structures are in place. If a severe storm hits the area, it may lead to erosion on site. Topsoil stockpiles may be prone to erosion due to lack of vegetation cover. Water control structures may fail or severe rainstorms may cause excessive run-off. Surface compaction due to activities taking place. |
| Probability | Very low |  |  |  |  |
| Significance | Low |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 3. SOIL |  |  |  |  |  |
| Nature of the impact | Potential of soil contamination. |  |  |  | None. |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Vehicle/equipment breakages and oil/lubricant /diesel spills may contaminate soil. |
| Probability | Moderate |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 3. SOIL |  |  |  |  |  |
| Nature of the impact | Loss of soil structure |  |  |  | None |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | In the process of removing topsoil the soil layers are mixed and the structure may be disturbed. |
| Probability | High |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 3.SOIL |  |  |  |  |  |
| Nature of the impact | Loss of soil fertility |  |  |  | None |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Short |  |  |  | The mixing of soil during site preparation, compaction and potential pollution (spillages form oil etc.) all may cause this situation. |
| Probability | Definite |  |  |  |  |
| Significance | Low |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 4.LAND CAPABILITY |  |  |  |  |  |
| Nature of the impact | Temporary loss of land capability to support grazing. The small area (0.5 ha) where the active mining activities occur (excavations, tailings dumps, stock piles, mining equipment) etc. will thus be temporary alienated, until the area is rehabilitated. All excavations would be rehabilitated as part of the mining process during which excavations are back-filled. The rest of the application area will still be used by the landowner as agricultural land. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Site preparation for additional mining sites and the construction, operation of listed infrastructure, the land capability of the active mining area will be totally destroyed. |
| Probability | Definite |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 5. LAND USE |  |  |  |  |  |
| Nature of the impact | The rehabilitation of the historically disturbed areas will have a positive impact on land use. This is a new mining operation on an old disturbed area and therefore will lose its land use to support grazing on a certain portion of the 5 ha during the next 2 years. Only a small portions of land (0.5 ha at a time) would be affected by the mining operation relation to the total mining right application area of 5 ha. All excavations would be rehabilitated as part of the mining process during which excavations are sloped. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long to permanent |  |  |  | Site preparation for mining and the construction, operation of listed infrastructure |
| Probability | Definite |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 6.VEGETATION |  |  |  |  |  |
| Nature of the impact | Vegetation clearance, disturbance and trampling. Destruction of habitats for vegetation. Due to a disturbed ecosystem, bare ground and spreading of exotics can follow. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | The site preparation for new sites, construction of listed infrastructure will cause destruction of habitats for vegetation. Due to a disturbed ecosystem, bare ground and invasion of exotics could further spread. The vegetation needs to be cleared to remove the topsoil. |
| Probability | Definite |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 6.VEGETATION |  |  |  |  |  |
| Nature of the impact | Habitat change, loss of species, spread of alien and invasive species. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Permanent |  |  |  | The change in the current habitat will be mitigated during final rehabilitation. |
| Probability | High |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 6.VEGETATION |  |  |  |  |  |
| Nature of the impact | Dust coverage of plants. |  |  |  | None |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Heavy trucks and other vehicles on dirt roads, stockpiling, dumping of tailings are mainly responsible for this impact. |
| Probability | High |  |  |  |  |
| Significance | Low |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 7. WILDLIFE |  |  |  |  |  |
| Nature of the impact | Wildlife or wildlife habitat destruction /change / disturbance. |  |  |  | None |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Permanent |  |  |  | The flora which normally serves as habitat for animals would be destroyed during site preparation. The increase in activity will temporarily scare other animals. The area will serve as a new habitat after rehabilitation. |
| Probability | Very High |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 7. WILDLIFE |  |  |  |  |  |
| Nature of the impact | Injury and death to wildlife. |  |  |  | None |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Short |  |  |  | The movement of vehicles may kill certain insects, rodents and possible birds. Most of the remaining animal life will however move away due to noise. |
| Probability | Very low |  |  |  |  |
| Significance | Low |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 7. WILDLIFE |  |  |  |  |  |
| Nature of the impact | Restoration of habitat. |  |  |  | None |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Short |  |  |  | As rehabilitation progresses the habitat of certain species will be restored/created (Closure objective) Animals will probably only move back when human movement is limited. |
| Probability | Low |  |  |  |  |
| Significance | Low |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 8. SURFACE WATER |  |  |  |  |  |
| Nature of the impact | Increased silt load. Clearing topsoil for footprint areas can increase infiltration rates of water to the groundwater system and decrease buffering capacity of soils to absorb contaminants from spills on surface. This can increase the risk of contamination of the groundwater system (increases aquifer vulnerability. |  |  |  |  |
| Extent | Local |  |  |  | Activity causing the impact |
| Duration | Short |  |  |  | The clearance of vegetation and the traffic on access roads will all contribute to an increase in the silt load on the mining area. |
| Probability | Moderate |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 8. SURFACE WATER |  |  |  |  |  |
| Nature of the impact | Change in surface water quality. Spillages from vehicles and also surface water run-off that is not adequately diverted away from the active mining excavations could end-up in the excavations creating problems regarding water quality and hindering the mining process. Surface run-off from active mining sites if not adequately contained on site could end-up in the adjacent undisturbed natural veld. If the natural surface run-off is not adequately diverted in the case of the dry-water course area, mining sections it could become silted-up. |  |  |  |  |
| Extent | Local |  |  |  | Activity causing the impact |
| Duration | Short |  |  |  | “Dirty / Clean” water systems at facilities like the overburden dumps, roads, excavations, etc. may impact on the quality of the surface water. The water should be contained in the surface runoff control measures provided therefore. |
| Probability | Moderate |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 8. SURFACE WATER |  |  |  |  |  |
| Nature of the impact | Change in surface water quantity: Notwithstanding the above-mentioned facts, it is not expected that mining operations will have any effect on the boundaries or the general water flow of the catchment. Standing water in trenches could as the result of rain/ surface run-off ending up in shallow depressions. Water for the dust suppression might be used from the borehole. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | It is an operational objective to contain or divert all surface run-offs from the active mining excavations area mainly due to pollution (sediment) potential. This will reduce the run-off quantity, although small in comparison with the drainage area in total. |
| Probability | High |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 9. GROUND WATER |  |  |  |  |  |
| Nature of the impact | Reduction of groundwater quality Mining activities are not likely to impact on local ground-water quality. No chemicals area used during the mining process. Handling of waste and transport of building material can cause various types of spills (domestic waste, pit latrines, hydrocarbons) which can infiltrate and contaminate of the groundwater system. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  |  |
| Probability | Definite |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| 9. GROUND WATER |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Nature of the impact | Even though abstraction is likely to have a minimal effect on the surrounding groundwater users, this is a new use, and groundwater levels are expected to continue current trends. Groundwater will be abstracted for potable water supply and dust suppression. The volume of water needed is small (2 000 Lit/hr) in comparison to other water use and will have a small impact on the surrounding aquifer. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Opencast mining operation. |
| Probability | Low |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 10. AIR QUALITY |  |  |  |  |  |
| Nature of the impact | Dust will be generated during the mining operation (loading with an excavator on to a dump truck) and transportation to the plant (screen & crushing plant) and on gravel/dirt/farm roads. The crushing of the gravel is a wet process and therefore minimum dust is generated. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Initial construction work with regard to infrastructure (roads) that involves earth moving equipment. Dust could be generated as indicated during mining. |
| Probability | Moderate |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 11. NOISE POLLUTION |  |  |  |  |  |
| Nature of the impact | Noise will be generated during the mining operation (loading with an excavator on to a dump truck) and transportation to the plant (screen & crushing plants). The mine itself is located in rural landscape. The impact would be of more importance regarding the direct worker environment that should adhere to the requirements in terms of the Mine Health and Safety Act. |  |  |  |  |
| Extent | Local |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Earth moving equipment and vehicles (trucks). |
| Probability | Definite |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 12. ARCHAEOLOGICAL AND CULTURAL SITES |  |  |  |  |  |
| Nature of the impact | The terrain is not archaeologically vulnerable as it was disturbed before. It is unlikely that the proposed development will result in any significant archaeological impact at the site. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Permanent |  |  |  |  |
| Probability | Definite |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X |  |  |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 13. SENSITIVE LANDSCAPE |  |  |  |  |  |
| Nature of the impact | No sensitive landscapes identified. |  |  |  |  |
| Extent | Not applicable |  |  |  | Activity causing the impact |
| Duration | Not applicable |  |  |  |  |
| Probability | Not applicable |  |  |  |  |
| Significance | Not applicable |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 14.VISUAL ASPECTS |  |  |  |  |  |
| Nature of the impact | Mining will be partly visible to the neighbours living there. The operation is not visible to from any tourist road. |  |  |  |  |
| Extent | Site |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Mining operation. |
| Probability | Definite |  |  |  |  |
| Significance | Low |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 15. SOCIO ECONOMICS |  |  |  |  |  |
| Nature of the impact | Increase in Socio – economic activity at local level. The project in itself would ensure that approximately 6 workers would be assured of a job for some time. Job creation plays a major role in increasing the economic well being of employees and their dependants in the Vereeniging district. Once all mining operations have ceased it would definitely have a negative impact. |  |  |  | The increase in socio-economic activity will add to the current growth and development in Vereeniging district already created by industry and mining. |
| Extent | Local |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  | Additional employment opportunities created. |
| Probability | Definite |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 15. SOCIO ECONOMICS |  |  |  |  |  |
| Nature of the impact | The main impact on the landowners is visual impact and the small area of 5 ha that will not be available for agricultural activities at any given time for 2 years. |  |  |  | The economic benefits in terms of investment and the delivery of services in the Gauteng province will get an additional benefit from the project. |
| Extent | Regional |  |  |  | Activity causing the impact |
| Duration | Very Long |  |  |  |  |
| Probability | High |  |  |  |  |
| Significance | Moderate |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

| ASPECT | IMPACTS |  |  |  | CUMULATIVE IMPACTS |
| --- | --- | --- | --- | --- | --- |
| 16. INTERESTED & AFFECTEDPARTIES |  |  |  |  |  |
| Nature of the impact | Impact of activities on I&AP’s Temporary loss of utilization of the mining focus areas for agricultural purposes. The long-term benefits far out-weight the current benefits from the current use. Loss of cattle due to falling of animals in mine workings if not fenced. No negative impact is expected that could be appropriately mitigated, such as the eventual rehabilitation of the excavations. |  |  |  |  |
| Extent | Local |  |  |  | Activity causing the impact |
| Duration | Long |  |  |  |  |
| Probability | High |  |  |  |  |
| Significance | High |  |  |  |  |
| Phase responsible for the impact | Phase 1 | Phase 2 | Phase 3 | Closure |  |
|  |  | X | X | X |  |

- Motivation where no alternative sites were considered.

Alternative is not applicable.  The current land use natural vegetation. The option to explore the possibility for mining is an alternative land use. The applicant, Raubex Construction (Pty) Ltd. is not interested in any other alternative land use over this land aside of mining of gravel or any other activity, or method use other than mining for the aforementioned minerals in the conversional way, which is the most cost effective.  Please note that no additional infrastructure will be established, and therefore no alternatives for the location of infrastructure were identified

- Statement motivating the alternative development location within the overall site. (Provide a statement motivating the final site layout that is proposed)

The application area applied for is only 5 hectares thus the development location is limited to this area and the area where the mineral deposits occur.

- FULL DESCRIPTION OF THE PROCESS UNDERTKEN TO IDENTIFY, ASSESS and rank the impacts and risks the activity will impose on the preferred site (In respect of the final site layout plan) through the life of the activity (Including (i) a description of all environmental issues and risks that are identified during the environmental impact assessment process and (ii) an assessment of the significance of each issue and risk and an indication of the extent to which the issue and risk could be avoided or addressed by the adoption of mitigation measures.)

See Table 11 below

Table 11: Technical & Management Action Plans

| Environmental Component | Geology |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| - No mitigation exists except to slope the excavations. - As mining progressed and the excavation has been sloped, a certain amount of overburden material and topsoil would be placed on these areas. This will not restore the geology, but will mitigate the impact. - Planned, systematic and thorough mining of the mineral resource (gravel deposits) should take place. - Optimal utilization of the mineral resource should take place within the boundaries of the mining terrain. - Strip, remove and store soil and overburden as far as practical in an orderly fashion and replace as far as possible on back-filled areas, in the reverse order once decision have been taken that no further mining would take place in a particular section or which might still be traversed by vehicles and disturbed in the process. Cognisance should be taken of the fact that bulk sampling would take place by means of an opencast mining method until such level is reach / cut-off point is reach where rehabilitation could begin. - Care must be taken that the removal of gravel deposits by means of earthmoving equipment is restricted to what is really necessary to achieve the objective. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Optimal exploration of the mineral resource in order to ensure to facilitate better rehabilitation planning. The overburden and topsoil (where available) must be replaced in a responsible and planned manner in order to achieve some conformity with the surrounding undisturbed area. |  |

| Environmental Component | Topography |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| - All trenches should be partly back-filled where possible with waste and eventually sloped and covered with a shallow layer of topsoil (if available). - Access to all active mining excavation areas should be controlled. The active mining area should be fenced off. The necessary warning signs should be put in place. All mining activities should be restricted to the fenced-off area. - Surface run-off control should be put in place at active trenches (preventing water from entering) and also rehabilitated tailings dumps and overburden dumps in order to prevent the loss of growth medium on top of the dumps. Mining would be done according to a definite MWP (only disturbing an area that is really necessary). As part of the MWP the handling of tailings material, overburden material, construction of dumps and back-filling of trenches should also form part of it. Rehabilitation of the new topographical landscape in such a way that it would blend in with the surrounding landscape and allow normal surface drainage to continue. As soon as a section of the mining site would not be explored anymore it should be rehabilitated (planned and phased manner). |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Rehabilitation of the new topographical landscape in such a way that it would blend in with the surrounding landscape and allow normal surface drainage to continue. Rehabilitation in such a way that the new landscape features would be stable and would not pose any safety hazard to human and animal anymore. |  |

| Environmental Component | Soil (topsoil & access roads) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Handling of topsoil as a natural resource: Any future expansion of the trenches or construction of infrastructure should be preceded by the removal of all available topsoil. The surface of any new areas to be disturbed must be kept to a minimum. All available topsoil/overburden material should be removed and stockpiled for rehabilitation purposes. Access roads, etc: The clearing of soil surface areas would be restricted to what is really necessary for the construction of infrastructure. Wherever possible all topsoil should be removed and stockpiled for rehabilitation purposes. Overburden material should also be stockpiled separately if practically possible. Topsoil and overburden material should be transported to an area earmarked for rehabilitation. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The topsoil removed in the site preparation process should be replaced during the rehabilitation exercise. |  |

| Environmental Component | Soil (soil compaction) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Soil compaction: The mining operation should only be restricted to what is really required (demarcated area of exploitation) within the fenced-off area. Access roads towards the sites would be restricted only to the roads (exiting farm roads & roads established in consultation with the surface owner). No land would be disturbed unnecessarily. Mining& rehabilitation should be done in a well-planned manner (according to a MP) and in the process ensuring that activities are only restricted to surface areas really required. Compaction of soil surface areas would be alleviated once rehabilitation of certain area starts. Certain roads would probably remain for access (in consultation with the surface owner). Those that would not be required would be ripped and rehabilitated. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Alleviation of compaction of soils would be done during rehabilitation of the mining terrain, including roads. |  |

| Environmental Component | Soil (Soil erosion) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Soil Erosion: To take preventive steps against land disturbance like erosion. Implement and maintain cut-off trenches/berms to prevent erosion. Re-vegetation of exposed soil surfaces (man-made surfaces on tailings dumps , overburden dumps, disturb surfaces in excavated sites, roads, etc) should happen as soon as a particular activity has ceased in order to act as a sufficient erosion prevention measure. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No soil erosion must be visible and no potential for soil erosion must be present at closure. |  |

| Environmental Component | Soil (Soil contamination) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Potential for soil contamination: Vehicles to be inspected to ensure no oil and hydraulic fluid leaks occur. All oil spills on soil to be removed and bio-remediate immediately (certain commercial products are available such as Terrasorb or it could be rehabilitated by means of the application of fertilizer and turn with a spade from time to time in order to enhance the natural occurring soil microbial activity). No servicing of vehicles must occur except on a concrete floor or over PVC lined area in an area allocated for that. Training w.r.t pollution hazards and their impact on the environment must be given as part of induction training. An incidence register for this purpose must be kept. Drip trays must be available and used where emergency repairs is done. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Closure can only be given once all soil contamination measures have been conducted to prevent and remediate any incidences. |  |

| Environmental Component | Soil (Soil structure) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Change in Soil structure: Ensure that all available (if any) topsoil is carefully removed in different areas. The soil must also be compacted as backfilling is done. No unnecessary driving outside the active mining area is allowed due to soil compaction that may occur. Use organic material e.g. manure to restore the soil structure during rehabilitation. Ensure that the rehabilitation plan makes provision for ripping of roads and spreading of organic material and that this is used during rehabilitation. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No compaction of any roads or any other area must be present during closure. If the soil structure is disturbed mitigation measures e.g. the use of organic material, lime and fertilizers must be implemented to restore the soil structure. |  |

| Environmental Component | Soil (Soil fertility) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Soil fertility: Little can be done to preserve the moisture status of the soil once it is exposed. The soil must be used for rehabilitation as quickly as possible. The soil on the rehabilitated area must be analysed to determine the deficiencies and fertilizer and lime must be ploughed into the soil to restore its fertility, if necessary. Ensure that stockpiled soil is kept clean and where possible ensure that the topsoil is treated with organic material and fertilized. Do not use stockpiled soil for any other purpose but for rehabilitation. Do not use topsoil to construct roads. Ensure the rehabilitation plan makes provision for fertiliser. Make sure rehabilitated topsoil is analyzed in a laboratory. The type of fertilizer would depend on a soil analyses and fertilizer recommendation. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The soil must be fertile enough to sustain vegetation. |  |

| Environmental Component | Land Capability |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| The disturbance of land must be restricted (kept to a minimum) to the planned fenced-off, active mining site only. Remove topsoil where it is available. Take care that roads needed are restricted to one entry to the area for mining purposes. If new land is used for roads to enter the area it must be done in consultation with the surface owner. All rehabilitation will be done according to the final rehabilitation plans after approval by the Department of Mineral Resources (DMR). Topsoil will be placed in areas where it was removed and the areas will be re-vegetated accordingly. Ensure that the rehabilitation plan is implemented. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Rehabilitated to the state that it is suitable for the predetermined and agreed land capability. |  |

| Environmental Component | Land Use |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| The disturbance of land must be restricted (kept to a minimum) to the planned active, fenced-off mining site only. Remove topsoil where it is available. Take care that roads are the only areas used to enter the area for mining purposes. If new land is used for roads to enter the area it must be done in consultation with surface owner. All rehabilitation will be done according to the final rehabilitation plans after approval by the Department of Mineral Resources (DMR). Topsoil will be placed in areas where it was removed and the areas will be re-vegetated accordingly. Ensure that the rehabilitation plan is implemented. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The opencast section requires the land to be totally disturbed. The replacement of tailings material, overburden and topsoil would ensure that the land is able to support some grazing. |  |

| Environmental Component | Vegetation |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| No mitigation exists except to replace the vegetation by reseeding of grasses and natural growth. Mining should be done in a well-planned manner (according to a MP) and in the process ensuring that activities are only restricted to surface areas really required. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| During rehabilitation indigenous vegetation cover comprising of local plant species should be established in order to ensure a well-adapted sustainable plant cover that would be able to prevent erosion of the replaced topsoil on the disturbed mining site exposed surfaces, tailings dumps, etc.). |  |

| Environmental Component | Vegetation |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Habitat change, loss of species, spread of alien and invasive species: No mitigation exists except to replace the vegetation by reseeding of grasses. Mining should be done in a well-planned manner (according to a MP) and in the process ensuring that activities are only restricted to surface areas really required. Develop and implement an invasive and alien control programme to control the spread of weeds and other invasive species. Eradicate exotic weeds and invader species if it invades the terrain. All illegal invader plants and weeds shall be eradicated as required in terms of Regulation 15 & 16 of the Act on Conservation of Agricultural Resources, 1983 (Act no. 43 of 1983) which list the plants. An invasive and alien control programme must be implemented by the mine. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No invasive and alien species must be present after closure. A post-closure control program must also be implemented. |  |

| Environmental Component | Vegetation |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Ensure that all roads on the mining site (utilized by mining vehicles) are daily sprayed with water to control dust. Site inspections to ensure the spraying are done. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No excessive dust must be present during the normal growth season after closure. |  |

| Environmental Component | Wildlife (habitat) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Wildlife or wildlife habitat destruction /change / disturbance : To take care that no new or unnecessary destruction of habitats, other than the demarcated mining site should take place. Restoration of habitat: Ensure the rehabilitation plan is implemented. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The animal life habitat must be restored after decommissioning. Success will be measured against the extent to which the animals return to the area. |  |

| Environmental Component | Wildlife (Injury and death) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Injury and death to wildlife: Re-establish trees and grass cover as soon as possible during and after mining. Fence area off to ensure that no person can enter without permission. Ensure that the rehabilitation plan is compiled and executed. Keep incidence register on killings and disturbances. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The animal life habitat must be restored after decommissioning. Success will be measured against the extent to which the animals return to the area. |  |

| Environmental Component | Wildlife |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Make game catching, traps, snares, poaching and any other unnecessary disturbance of animals a disciplinary offence. All staff must undergo basic environmental awareness lecture during induction training. Machine operators and drivers to undergo appropriate level of environmental impact training to ensure they understand their impact on the environment. Ensure all staff working on the opencast section undergo basic lecture during induction phase. Introduce the actions as listed above into disciplinary code as offence. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The post-closure phase must be suitable for further restoration of the newly man-made animal habitat. The area must be stable and acceptable for the return of animal- and plant life. |  |

| Environmental Component | Surface Water (quality) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Change in surface water quality: Storm water control measures must be implemented to divert clean water away from the active mining site and keep contaminated water contained. Water control structures must be well designed and constructed to ensure a minimum down wash of topsoil. Vegetation disturbance must be as little as possible. The MWP must be strictly adhered to. Re-vegetation to be done as quickly as possible. Final re-vegetation to be done as per rehabilitation plan. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The post closure water run-off may in no circumstance impact negatively on the water quality. |  |

| Environmental Component | Surface Water (quantity) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Change in surface water quantity: Once the area is rehabilitated the surface run-off will be restored and normal clean water run-off will end-up in the drainage system. Once the area is rehabilitated the normal surface run-off drainage will be restored according to rehabilitation plan. The disturbed surface area must be rehabilitated to ensure some normal drainage. Minimal run-off should end-up in trenches. Final rehabilitation will be done according to the final rehabilitation plans after approval by the Department of Mineral Resources. The depth of the operation of maximum of 5m will not intersect the groundwater table thus no negative impact. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Ultimately rehabilitation of the disturbed mining site and the construction of run-off control structures in a planned and phased manner would ensure normal drainage and stability of rehabilitated site. The drainage must be away from the N4 tar road. |  |

| Environmental Component | Ground Water (quality) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Reduction of groundwater quality: Storm water control measures must be implemented to divert clean water away from the site and keep (silt) contaminated water contained. Vehicles to be inspected to ensure no oil and hydraulic fluid leaks occur. All oil spills on soil to be removed and bio-remediate immediately. No servicing of vehicles must occur except at the workshops. Training w.r.t pollution hazards and their impact on the environment must be given as part of induction training. Storage of fuel and oil should be done according to best practices, within a bunded area and in containers of which the integrity is sound. The mining processes will not introduce any harmful or toxic substances and the most likely sources of pollution to the groundwater system would be associated with the infrastructure and / or workshop area. The most likely contaminants is therefore nitrate and bacteria (from sewage / pit latrines), as well as hydrocarbons (from vehicle accidents, diesel storage and the workshop area). An incidence register for this purpose must be kept. Drip trays must be available and used where emergency repairs is done. All waste must be stored according to best practices and disposed at an authorized waste disposal facility. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Post water quality need to indicate a positive trend/improvement. |  |

| Environmental Component | Ground Water (quantity) |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Reduction of groundwater quantity, lowering of groundwater level: Water levels in the boreholes that are used for mining activities should be recorded monthly. Water volumes should be recorded continuously to ensure compliance with the water use authorization for abstraction. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Post water quality need to indicate a positive trend/improvement. |  |

| Environmental Component | Air Quality |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Dust: The mining method will serve as mitigation measure because mining will limit dust to the active mining area (area where the excavator and the trucks are operating). Daily spraying of roads with water. Inspection should be done on a daily basis. If new roads are constructed, in coordination with surface owner, dust pollution must be mitigated by means of spraying the roads with water. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Dust count must be the same as before mining. Rehabilitation of the bulk sampling site would ensure that no dust is generated from exposed surfaces. |  |

| Environmental Component | Noise |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Ensure the required silencers are placed on all engines and compressors. No mitigation to reverse hooters is allowed due to safety standards. Inspection of vehicles and machinery to ensure silencers are fitted. Ensure that a complaints register is created, managed and maintained. Vehicles and earthmoving equipment should be equipped with the necessary silencers and regularly maintained in a good working condition. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No noise attributed to mining will be generated from the site after closure anymore. During decommissioning and closure phase some earth moving equipment and trucks would be utilized for rehabilitation. |  |

| Environmental Component | Archaeological and Cultural Sites |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| All grave yard needs to be avoided if found However, the potential occurrence of unmarked graves or subsurface finds not recorded during this survey can never be excluded, so it is advised that SAHRA and a qualified archaeologist are informed immediately if archaeological objects are uncovered. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No site of archaeological importance should be disturbed or damaged until the necessary permit from SAHRA has been issued. |  |

| Environmental Component | Sensitive Landscapes |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| None |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |

| Environmental Component | Visual Aspects |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Visual impact would be addressed by means of; * re-vegetation of disturbed areas with grasses; * removal of any temporary building, scrap, domestic waste, etc. that would otherwise contribute to a negative visual impact. Concurrent rehabilitation should be done simultaneously as mining activities progress. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| No residual visual impacts will remain after closure. The terrain should blend in with the surrounding landscape. |  |

| Environmental Component | Socio-Economics |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| There will be a very small increase in Socio – economic activity at local level, because of the size of this mining activity. |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| The economic development must deliver a multiplier effect that will contribute to the local economy long after closure. |  |

| Environmental Component | Interested and Affected Parties |
| --- | --- |
| Environmental Management/Mitigation Measures/Action Plans/Commitments |  |
| Access control should always be a priority. Active mining site should be fenced off and also any deep water holes. If any problem should arise, meetings will be held with the landowners and affected parties to consult them on certain matters like permission to prospect and pollution. No mining should be conducted under or near Eskom power line (10 m distance should be kept) (Permission of Inspector of Mines should be obtained.) |  |
| EMP Performance Assessment & Monitoring Reporting |  |
| To be included in EMP/EIA. |  |
| Closure Objective |  |
| Not to be an economic, social or environmental liability to the local community or the state now or in the future. The company will ensure that the interest of all interested and affected parties will be considered. |  |

- Assessment of each identified potentially significant impact and risk

(This section of the report must consider all the known typical impacts of each of the activities (including those that could or should have been identified by knowledgeable persons) and not only those that were raised by registered interested and affected parties).

Table 12: Identified Potentially Significant Impacts & Risks

| NAME OF ACTIVITY (E.g. For prospecting - drill site, site camp, ablution facility, accommodation, equipment storage, sample storage, site office, access route etc…etc…etc E.g. For mining ,- excavations, blasting, stockpiles, discard dumps or dams, Loading, hauling, and transport, Water supply dams and boreholes, accommodation, offices, ablution, stores, workshops, processing plant, storm water control, berms, roads, pipelines, power lines, conveyors, etc…etc…etc… | POTENTIAL IMPACT (Including the potential impacts for cumulative impacts) (e.g. dust, noise, drainage, surface disturbance, fly rock, surface water contamination, groundwater contamination, air pollution etc…etc…) | ASPECTS AFFECTED | PHASE In which impact is anticipated | SIGNIFICANCE if not mitigated | MITIGATION TYPE (modify, remedy, control, or stop) through (e.g. noise control measures, stormwater control, dust control, rehabilitation, design measures, blasting controls, avoidance, relocation, alternative activity etc. etc) | SIGNIFICANCE if mitigated |
| --- | --- | --- | --- | --- | --- | --- |
| Excavations for gravel, stone aggregate and dolomite | 1.1 Removal of the grave/stone up to 4 m. Disturbance of 0.5 hectare at any given time. | Geology & soil | Operational | High - | The bulk of the material mined will be sold. The impact will be mitigated by sloping the sides and stabilizing the soil to prevent erosion | Low + |
|  | 1.2 Change in landform. The entire mining area will be lowered by 4 m and normal surface drainage will be disturbed at this specific point. The pit will not be backfilled | Topography | Operational and closure | Moderate - | The pit will not be backfilled. The sides will be sloped and top soiled and vegetated. A surface water cut-off trench should be put in place around the active mining site in order to prevent surface run-off water on the mining site. Rehabilitation of the new sloped landscape in such a way that it would blend in with the surrounding landscape. | Moderate + |
|  | 1.3 Stripping of all available topsoil and stockpiled. Stockpile and plant area of 0.5 hectares at any given time. | Soil | Construction and Operational | Low - | Any area on the mining area where disturbance will take place the top soil must be removed and stockpiled for rehabilitation purposes in a demarcated area. | Low + |
|  | 1.4 Soil erosion: Due to the fact that certain surface areas would become devoid of any vegetation cover and compacted this would lead to lesser infiltration of rain water and more run-off that could cause erosion on bare disturbed areas and side slopes | Soil | Construction | Low- | To take preventive steps against erosion. Implement and maintain cut-off trenches and or berms around the mining area to prevent water entering that can cause erosion. Concurrent rehabilitation and re-vegetation of mined areas must happen as soon as the particular area is mined out. Rehabilitated areas must be inspected and managed in such a way that any signs of erosion can be mitigated immediately. | Low + |
|  | - Land capability and land use. Loss of land to support grazing. | Land capability & Land use | Operational and closure | Low- | As this is only a very small area of 5 hectares, the impact is not so big. As the sides will be sloped and vegetated the rehabilitated area must be treated as sensitive when grazed as overgrazing can trigger erosion and infiltration of declares weeds. |  |
|  | 1.6 Generation of dust by excavating crushing and vehicle movement | Air quality | Operational | Low - | The mining method will serve as mitigation measure because it will limit dust to the active mining area, where the excavator and trucks operating. Daily spraying of the roads with water. |  |

- Summary of specialist reports.

(This summary must be completed if any specialist reports informed the impact assessment and final site layout process and must be in the following tabular form)

Table 13: Specialist Reports

| LIST OF STUDIES UNDERTAKEN | RECOMMENDATIONS OF SPECIALIST REPORTS | SPECIALIST RECOMMENDATIONS THAT HAVE BEEN INCLUDED IN THE EIA REPORT (Mark with an X where applicable) | REFERENCE TO APPLICABLE SECTION OF REPORT WHERE SPECIALIST RECOMMENDATIONS HAVE BEEN INCLUDED. |
| --- | --- | --- | --- |
| None |  |  |  |

- Environmental impact statement

- Summary of the key findings of the environmental impact assessment;

The small scale gravel, stone aggregate and dolomite mining operation is definitely going to have an impact on the environment.

The main impact relates to topography, geology, soil, vegetation, and land use and land capability.

The gravel resource will be mined over a period of 2 years or less.

The existing land-use natural vegetation.

This is a small operation and for the next 2 years only a small portion of the farm will be temporarily alienated.

The conservation of topsoil is of utmost importance and therefore in order to ensure a sustainable land use again on the 5 ha, the top at least 30 cm topsoil where available needs to be removed prior to mining of the underlying gravel (up to 4 m depth). This will be used again as growth medium during the rehabilitation phase of the quarry. Topsoil will be stored in berm walls on the border of the quarry in order to divert any surface run-off during a rainfall event.

Other environmental impacts relates to the day to day operation that could easily be managed, such as dust and noise.

- Final Site Map

Provide a map at an appropriate scale which superimposes the proposed overall activity and its associated structures and infrastructure on the environmental sensitivities of the preferred site indicating any areas that should be avoided, including buffers.

See Appendix 1(b).

- Summary of the positive and negative impacts and risks of the proposed activity and identified alternatives;

The loss of land use and land capability will be temporary as the site will be rehabilitated in such a way that it allows the establishment of a grass cover again, but because of pre-existing mining activities it have probably not been used at all. The rest of the farm will still be continued to be used for grazing for game.

Although this is a small gravel mining operation it would also add to the increased economic activity within the farming and exiting mining community around Ennerdale.  Jobs for 6 permanent labourers will be created.

Negative impacts on the area are expected to be temporary and can be mitigated to a large extent if the recommendations of the EMP are adhered to e.g. rehabilitation.

No concerns have been raised as yet by any I & AP.

The specific occurrence of the gravel deposit dictates the selection of the specific mining site.

- Proposed impact management objectives and the impact management outcomes for inclusion in the EMPr;

Based on the assessment and where applicable the recommendations from specialist reports, the recording of proposed impact management objectives, and the impact management outcomes for the development for inclusion in the EMPr as well as for inclusion as conditions of authorisation.

The main closure objective of 	the applicant is to rehabilitate the entire mining site in such a way to ensure that the man-made topographical landscape (which is pre-existing) would be rehabilitated toward agricultural use and to blend in with the surrounding landscape, not pose a safety hazard to humans and animals, while at the same time allow for alternative land uses. Establish a self-sustaining and stable vegetation cover in order to mitigate the visual impact, to control erosion and to create some habitat for animals. The rehabilitated environment also needs to be aesthetically acceptable according to the principle of BPEO.  The applicant will ensure that the Operation/Sites are:

- Neither a danger to public health and safety nor to animal health and safety;

- Not a source of any pollution;

- Stable (ecological and geophysical);

- Rehabilitated to the state that is suitable for the predetermined and agreed land use (grazing);

- Compatible with the surrounding biophysical environment;

- A sustainable environment;

- Aesthetically acceptable;

- Not an economic, social or environmental liability to the local community or the state now or in the future.

- Aspects for Inclusion as Conditions of Authorisation.

Any aspects which must be made conditions of the Environmental Authorisation

None

- Description of Any Assumptions, Uncertainties and Gaps in Knowledge.

(Which relate to the assessment and mitigation measures proposed)

None

- Reasoned Opinion As To Whether The Proposed Activity Should Or Should Not Be Authorised

- Reasons why the activity should be authorized or not.

This activity will have only low and very low impacts and no significant impacts were identified. No concerns were raised by the interested parties. These mining activities will have no significant impacts on them or their surrounding environment.

- Conditions that must be included in the authorisation

None

- Period for which the environmental authorisation is required.

24 months for initial permit period, thus 24 months in total.

- Undertaking

Confirm that the undertaking required to meet the requirements of this section is provided at the end of the EMPr and is applicable to both the Basic assessment report and the Environmental Management Programme report.

The undertaking will be part of the EMPR.

- Financial Provision

State the amount that is required to both manage and rehabilitate the environment in respect of rehabilitation.

R78 693.00 for rehabilitation. See quantum attached as Appendix 3.

- Explain how the aforesaid amount was derived.

The amount was determined through the quantum tables provided by DMR.

- Confirm that this amount can be provided for from operating expenditure

(Confirm that the amount, is anticipated to be an operating cost and is provided for as such in the Mining work programme, Financial and Technical Competence Report or Mining Work Programme as the case may be).

Yes it is hereby confirmed that the amount will be provided from operating expenditure.

- Specific Information required by the competent Authority

- Compliance with the provisions of sections 24(4)(a) and (b) read with section 24 (3) (a) and (7) of the National Environmental Management Act (Act 107 of 1998). the EIA report must include the:-

- Impact on the socio-economic conditions of any directly affected person.

(Provide the results of Investigation, assessment, and evaluation of the impact of the mining, bulk sampling or alluvial diamond prospecting on any directly affected person including the landowner, lawful occupier, or, where applicable, potential beneficiaries of any land restitution claim, attach the investigation report as an Appendix  .

The applicant has an agreement with the landowner and the landowner will be paid for the land used by mining when the activity starts. No other person will be directly affected by this activity.

- Impact on any national estate referred to in section 3(2) of the National Heritage Resources Act

(	Provide the results of		Investigation, assessment, and evaluation of the impact of the mining, bulk sampling or alluvial diamond prospecting on any national estate referred to in section 3(2) of the National Heritage Resources Act, 1999 (Act No. 25 of 1999) with the exception of the national estate contemplated in section 3(2)(	i)(vi) and (vii) of that Act, attach the investigation report as 	Appendix 2.19.2	 and confirm that the applicable mitigation is reflected in 2.5.3; 2.11.6.and 2.12.herein).

This activity will have no impact on archaeological structures.

- Other matters required in terms of sections 24(4)(a) and (b) of the Act.

(the EAP managing the application must provide the competent authority with detailed, written proof of an investigation as required by section 24(4)(b)(i) of the Act and motivation if no reasonable or feasible alternatives, as contemplated in sub-regulation 22(2)(h), exist. The EAP must attach such motivation as Appendix 1).

None

PART B

ENVIRONMENTAL MANAGEMENT PROGRAMME REPORT

- Draft environmental management programme.

- Details of the EAP

(Confirm that the requirement for the provision of the details and expertise of the EAP are already included in PART A, section 1(a) herein as required).

The EAP Mr. Daan Erasmus has a National Diploma in Agriculture Resource Utilization and a Baccalaureus Technologiae degree in Agricultural Extension.

Yes see Part A.

- Description of the Aspects of the Activity

(Confirm that the requirement to describe the aspects of the activity that are covered by the draft environmental management programme is already included in PART A, section (1)(h) herein as required).

Yes see Part A.

- Composite Map

(Provide a map (Attached as an Appendix 1 (b)) at an appropriate scale which superimposes the proposed activity, its associated structures, and infrastructure on the environmental sensitivities of the preferred site, indicating any areas that any areas that should be avoided, including buffers)

See Appendix 1(b).

- Description  of  Impact  management  objectives  including  management statements

- Determination of closure objectives

(ensure that the closure objectives are informed by the type of environment described)

The main closure objective of 	the applicant is to rehabilitate the entire mining site in such a way to ensure that the man-made topographical landscape would blend in with the surrounding landscape, not pose a safety hazard to humans and animals, while at the same time allow for alternative land uses. Establish a self-sustaining and stable vegetation cover in order to mitigate the visual impact, to control erosion and to create some habitat for animals. The rehabilitated environment also needs to be aesthetically acceptable according to the principle of BPEO.  Another main objective is to manage the surface water in such way that an acceptable water standard is achieved when a closure certificate is issued.

Raubex Construction (Pty) Ltd will ensure that the Operation/Sites are:

- Neither a danger to public health and safety nor to animal health and safety;

- Not a source of any pollution;

- Stable (ecological and geophysical);

- Rehabilitated to the state that is suitable for the predetermined and agreed land use;

- Compatible with the surrounding biophysical environment;

- A sustainable environment;

- Aesthetically acceptable;

- Not an economic, social or environmental liability to the local community or the state now or in the future.

Raubex Construction (Pty) Ltd will furthermore:

- ensure that the physical and chemical stability of the rehabilitated  site will be such that risk to the environment is not increased by naturally occurring forces to the extent that such increased risk cannot be contended with by the installed measures;

- subscribe to the optimal exploitation and utilization of South Africa’s mineral resources (Gravel);

- ensure that the mining  site is closed efficiently and cost effectively.

- ensure that the operation is not abandoned but closed in accordance with the relevant requirements;

- ensure that the interest of all interested and affected parties will be considered;

- ensure that the all-relevant legislation regarding mine closure will be adhered to, and all relevant application procedures followed.

- Volumes and rate of water use required for the operation

2000 litres a day will be used for dust suppression.

- Has a water use licence been applied for?

N/A.

- Impacts to be mitigated in their respective phases

Table 14: Measures to rehabilitate the environment affected by the undertaking of any listed activity

| ACTIVITIES (E.g. For prospecting - drill site, site camp, ablution facility, accommodation, equipment storage, sample storage, site office, access route etc…etc…etc E.g. For mining,- excavations, blasting, stockpiles, discard dumps or dams, Loading, hauling and transport, Water supply dams and boreholes, accommodation, offices, ablution, stores, workshops, processing plant, storm water control, berms, roads, pipelines, power lines, conveyors, etc…etc…etc.) | PHASE (of operation in which activity will take place. State; Planning and design, Pre-Construction, Construction, Operational, Rehabilitation, Closure, Post closure). | SIZE AND SCALE of disturbance (volumes, tonnages and hectares or m²) | MITIGATION MEASURES (describe how each of the recommendations in herein will remedy the cause of pollution or degradation and migration of pollutants) | COMPLIANCE WITH STANDARDS (A description of how each of the recommendations herein will comply with any prescribed environmental management standards or practices that have been identified by competent Authorities) | IME PERIOD FOR IMPLEMENTATION Describe the time period when the measures in the environmental management programme must be implemented Measures must be implemented when required. With regard to Rehabilitation specifically this must take place at the earliest opportunity.With regard to Rehabilitation, therefore state either: Upon cessation of the individual activity or, Upon the cessation of mining, bulk sampling or alluvial diamond mining as the case may be. |
| --- | --- | --- | --- | --- | --- |
| - Excavations | Operational | 6000 m³a month and 0.3 hectares at any stage | Concurrent rehabilitation by sloping the sides of the excavation to be stable/sustainable and covered with topsoil and vegetate. | Sides of pit to be sloped to 14 degrees for stability and providing a base for the replacement of topsoil. | As part of concurrent rehabilitation. |
| - Gravel Stockpile area | Operational | 0.2 hectares at any stage | Keep this area as small as possible within the demarcated area. Prevent spillages of fuels by machines | Immediate cleaning of spillages | Concurrent with mining |
| - Screening and crushing | Operational | 0.2 hectares at any stage | Keep this area as small as possible. Prevent spillages of fuels by equipment. | Immediate cleaning of spillages | Concurrent with the mining |

- Impact Management Outcomes

(A description of impact management outcomes, identifying the standard of impact management required for the aspects contemplated in paragraph)

| ACTIVITY (whether listed or not listed) (E.g. Excavations, blasting, stockpiles, discard dumps or dams, Loading, hauling and transport, Water supply dams and boreholes, accommodation, offices, ablution, stores, workshops, processing plant, storm water control, berms, roads, pipelines, power lines, conveyors, etc…etc…etc.). | POTENTIAL IMPACT (e.g. dust, noise, drainage surface disturbance, fly rock, surface water contamination, groundwater contamination, ait pollution etc…etc…) | ASPECTS AFFECTED | PHASE In which impact is anticipated (e.g. Construction, commissioning, operational, Decommissioning, closure, post-closure) | MITIGATION TYPE (modify, remedy, control, or stop) through (e.g. Noise control measures, storm water control, dust control, rehabilitation, design measures, blasting controls, avoidance, relocation, alternative activities. etc), E.g.  Modify through alternative method.  Control through noise control  Control through management and monitoring  Remedy through rehabilitation. | STANDARD TO BE ACHIEVED (Impact avoided, noise levels, dust levels, rehabilitation standards, end use objectives) etc. |
| --- | --- | --- | --- | --- | --- |
| - Excavations for gravel and stone | 1.1 Removal of the gravel up to -4m | Geology & soil | Operational | The bulk of the material removed will be sold. The impact will be mitigated be sloping the sides of the excavation and stabilizing the soil to prevent soil erosion. | Stable slopes that can sustain erosion without excessive erosion. |
|  | - Change in landform. The entire mining area will be lowered by 4m and normal surface drainage will be disturbed at this specific point. The pit will not be backfilled but will be sloped. | Topography | Operational and closure | The sides of pit will be sloped and the soil stabilized to prevent erosion. A surface water cut-off trench should be put in place around the active mining site I order to prevent surface water on the mining site. Rehabilitation of the new sloped landscape in such a way that it would blend in with the surrounding landscape. | Gentle stable slopes. |
|  | 1.3 Stripping of all available topsoil and stockpiled | Soil | Construction and operational | The top soil must be removed before any disturbance take place. The top soil must be removed and stockpile in a demarcated area for rehabilitation purposes. | Enough topsoil for rehabilitation to ensure sustainable vegetation. |
|  | 1.4 Soil erosion due to the fact that certain surface areas would become devoid of any vegetation cover and compacted. This would lead to lesser infiltration of rain water and more run-off that could cause erosion on bare disturbed areas and side slopes. | Soil | Construction and operational | To take preventive steps against erosion. Implement and maintain cut-off trenches and or berms around the mining area to prevent water entering that can cause excessive erosion. | No excessive erosion that cannot be stabilized. |
|  | 1.5. Loss of Land capability & land use. | Land capability & land use | Operational and closure | As this is only a very small area of 5 hectares, the impact is low. As the sides will be sloped and vegetated, the rehabilitated area must be treated as sensitive when grazed as overgrazing can trigger erosion and infiltration of declared weeds. | Sustainable rehabilitated area. |
|  | 1.6 Generation of dust by excavating, crushing/screening and vehicle movement | Air quality | Operational | The generation of dust will only be localized at the mining site. Daily spraying of roads with water | No excessive dust that can be harmful to the environment and humans. |

- Impact Management Actions

(A description of impact management actions, identifying the manner in which the impact management objectives and outcomes contemplated in paragraphs (c) and (d) will be achieved).

| ACTIVITY (whether listed or not listed) (E.g. Excavations, blasting, stockpiles, discard dumps or dams, Loading, hauling and transport, Water supply dams and boreholes, accommodation, offices, ablution, stores, workshops, processing plant, storm water control, berms, roads, pipelines, power lines, conveyors, etc…etc…etc.). | POTENTIAL IMPACT (e.g. dust, noise, drainage surface disturbance, fly rock, surface water contamination, groundwater contamination, ait pollution etc…etc…) | MITIGATION TYPE (modify, remedy, control, or stop) Through (e.g. noise control measures, stormwater control, dust control, rehabilitation, design measures, blasting controls, avoidance, relocation, alternative activities. etc) E.g.  Modify through alternative method.  Control through noise control  Control through management and monitoring  Remedy through rehabilitation. | TIME PERIOD FOR IMPLEMENTATION Describe the time period when the measures in the environmental management programme must be implemented Measures must be implemented when required. With regard to Rehabilitation specifically this must take place at the earliest opportunity.. With regard to Rehabilitation, therefore state either:- Upon cessation of the individual activity or Upon the cessation of mining bulk sampling or alluvial diamond prospecting as the case may be. | COMPLIANCE WITH STANDARDS (A description of how each of the recommendations in 2.11.6 read with 2.12 and 2.15.2 herein will comply with any prescribed environmental management standards or practices that have been identified by Competent Authorities) |
| --- | --- | --- | --- | --- |
| Excavations for gravel and stone. | 1.1 Removal of the gravel up to 4 m | The bulk of the material removed will be sold. The impact will be mitigated be sloping the sides of the excavation and stabilizing the soil to prevent soil erosion. |  |  |
|  | - Change in landform. The entire mining area will be lowered by 4 m and normal surface drainage will be disturbed at this specific point. The pit will not be backfilled | The side of pit will be sloped and the soil stabilized to prevent erosion. A surface water cut-off trench should be put in place around the active mining site I order to prevent surface water on the mining site. Rehabilitation of the new sloped landscape in such a way that it would blend in with the surrounding landscape. |  |  |
|  | 1.3 Stripping of all available topsoil and stockpiled | The top soil must be removed before any disturbance take place. The top soil must be removed and stockpile in a demarcated area for rehabilitation purposes |  |  |
|  | 1.4 Soil erosion due to the fact that certain surface areas would become devoid of any vegetation cover and compacted. This would lead to lesser infiltration of rain water and more run-off that could cause erosion on bare disturbed areas and side slopes. | To take preventive steps against erosion. Implement and maintain cut-off trenches and or berms around the mining area to prevent water entering that can cause excessive erosion. |  |  |
|  | 1.5 Loss of Land capability & land use | As this is only a very small area of 5 hectares, the impact is low. As the sides will be sloped and vegetated, the rehabilitated area must be treated as sensitive when grazed as overgrazing can trigger erosion and infiltration of declared weeds. |  |  |
|  | 1.6 Generation of dust by excavating, crushing/screening and vehicle movement | The generation of dust will only be localized at the mining site. Daily spraying of roads with water |  |  |

- Financial Provision

- Determination of the amount of Financial Provision

- Describe  the  closure  objectives  and  the extent  to  which  they  have  been  aligned  to  the baseline  environment  described  under  the Regulation

The main closure objective of 	the applicant is to rehabilitate the entire mining site in such a way to ensure that the new man-made topographical landscape would blend in with the surrounding landscape, not pose a safety hazard to humans and animals, while at the same time allow for alternative land uses. Establish a self-sustaining and stable vegetation cover in order to mitigate the visual impact, to control erosion and to create some habitat for animals. The rehabilitated environment also needs to be aesthetically acceptable according to the principle of BPEO.  Another main objective is to manage the surface water in such way that an acceptable water standard is achieved when a closure certificate is issued.

Raubex Construction (Pty) Ltd will ensure that the Operation/Sites are:

- Neither a danger to public health and safety nor to animal health and safety;

- Not a source of any pollution;

- Stable (ecological and geophysical);

- Rehabilitated to the state that is suitable for the predetermined and agreed land use;

- Compatible with the surrounding biophysical environment;

- A sustainable environment;

- Aesthetically acceptable;

- Not an economic, social or environmental liability to the local community or the state now or in the future.

Raubex Construction (Pty) Ltd will furthermore:

- ensure that the physical and chemical stability of the rehabilitated  site will be such that risk to the environment is not increased by naturally occurring forces to the extent that such increased risk cannot be contended with by the installed measures;

- subscribe to the optimal exploitation and utilization of South Africa’s mineral resources (SAND);

- ensure that the mining  site is closed efficiently and cost effectively.

- ensure that the operation is not abandoned but closed in accordance with the relevant requirements;

- ensure that the interest of all interested and affected parties will be considered;

- ensure that the all-relevant legislation regarding mine closure will be adhered to, and all relevant application procedures followed.

- Confirm  specifically  that  the  environmental objectives  in  relation  to  closure  have  been consulted  with  landowner  and  interested  and affected parties

Yes, the disturbance that will take place and the rehabilitation thereof were discussed on the site visit with the landowner.

- Provide  a  rehabilitation  plan  that  describes  and  shows  the  scale  and  aerial  extent  of  the  main mining activities, including the anticipated mining area at the time of closures.

##### Rehabilitation:

The clearing of soil surface areas would be restricted to what is really necessary for the construction of infrastructure/crushing plant.  During rehabilitation of these sites, or where vegetation is lacking or compacted, the areas would be ripped or ploughed and levelled in order to re-establish a growth medium and if necessary appropriately fertilised to ensure the regrowth of vegetation and the soil ameliorated based on a fertilizer recommendation (soil sample analysed).

##### Rehabilitation of access roads

- Whenever a mining permit is suspended, cancelled or abandoned or if it lapses and the holder does not wish to renew the permit or right, any access road or portions thereof, constructed by the holder and which will no longer be required by the landowner/tenant, shall be removed and/or rehabilitated to the satisfaction of the Regional Manager.

- Any gate or fence erected by the holder which is not required by the landowner/tenant, shall be removed and the situation restored to the pre-mining situation.

- Roads shall be ripped or ploughed, and if necessary, appropriately fertilised (based on a soil analysis) to ensure the regrowth of vegetation.  Imported road construction materials which may hamper regrowth of vegetation must be removed and disposed of in an approved manner prior to rehabilitation.

- If a reasonable assessment indicates that the re-establishment of vegetation is unacceptably slow, the Regional Manager may require that the soil be analysed and any deleterious effects on the soil arising from the mining operation, be corrected and the area be seeded with a seed mix to the Regional Manager’s specification.

#### Rehabilitation of the surface mining site

On completion of operations, all buildings, structures or objects on the camp/office site shall be dealt with in accordance with section 44 of the Mineral and Petroleum Resources Development Act, 2002 (Act 28 of 2002), which states:

(1)  When a prospecting right, mining right, retention permit or mining permit lapses, is cancelled or is abandoned or when any prospecting or mining operation comes to an end, the holder of any such right or permit may not demolish or remove any building, structure, object -

(A & B)  which may not be demolished in terms of any other law;

(C)	which has been identified in writing by the Minister for purposes of this section; or

(c)	which is to be retained in terms of an agreement between the holder and the owner or occupier of the land, which agreement has been approved by the Minister in writing.

(2)  The provision of subsection (1) does not apply to bona fide mining equipment which may be removed

The quarry surface area shall  be ripped or ploughed to a depth of at least 300mm and the topsoil previously stored adjacent the site, shall be spread evenly to its original depth over the whole  area.

After all the foreign matter has been removed from the mining sites, the side slopes and the quarry floor area will be sloped and levelled and the previously stored topsoil replaced.

The area shall then be fertilised if necessary (based on a soil analysis). The site shall be seeded with a vegetation seed mix (section C) adapted to reflect the local indigenous flora. Where the site has been rendered devoid of vegetation/grass or where soils have been compacted owing to traffic, the surface shall be scarified or ripped.

Photographs of the site, before and during the mining operation and after rehabilitation, shall be taken at selected fixed points and kept on record for the information of the Regional Manager.

Rehabilitation of the new topographical landscape in such a way that it would blend in with the surrounding landscape and allow normal (controlled) surface drainage to continue.

Implement water control systems in order to prevent erosion. Seed the area (see C. (below) for recommended seed mixture).

Visual impact would be addressed by means of;

- revegetation (grasses);

- removal of any building, scrap, domestic waste, etc. that would otherwise contribute to a negative visual impact.

##### Fertilising of Areas to be Rehabilitated

If a reasonable assessment indicates that the re-establishment of vegetation is unacceptably slow, the Regional Manager may require that the soil be analysed and any deleterious effects on the soil arising from the mining operation be corrected and the area be seeded with a seed mix to his or her specification.

##### Seeding of Grass Seed Mixture and planting of Woody Species

The eventual seed mixture takes into account the availability of seed, different soil situations and the prevailing climatic conditions of the area.  The following mixture will be applicable to the mining permit site:

Cenchrusciliaris

Cynodondactylon

Digitariaeriantha

Heteropogoncontortus

Panicum maximum

  - Demolition of infrastructure/buildings

On completion of operations, all buildings, structures or other  on the mining terrain shall be dealt with in accordance with section 44 of the Mineral and Petroleum Resources Development Act,2002 (Act No. 28 of 2002). There will be no permanent buildings.

  - Invasive and alien control programme

Develop and implement an invasive and alien control programme to control the spread of weeds and other invasive species. Eradicate exotic weeds and invader species if it invades the terrain.  All illegal invader plants and weeds shall be eradicated as required in terms of Regulation 15 & 16 of the Act on Conservation of Agricultural Resources, 1983 (Act no. 43 of 1983) which list the plants.

- Explain  why  it  can  be  confirmed  that  the rehabilitation plan is compatible with the closure objectives

There will be no excavations of processing areas that can cause after mining impacts or residues. It will only be the rehabilitation of the drilling sites.

- Calculate and state the quantum of the financial provision required to manage and rehabilitate the environment  in  accordance  with  the  applicable guideline

R78 693.00

- Confirm that the financial provision will be provided as determined

The financing for this project will be done from the account Raubex Construction (Pty) Ltd. the applicant himself out of own funds.

The guarantee will be provided in the form of Bank Guarantee after confirmation of the amount.

- Mechanisms for monitoring compliance with and performance assessment against the environmental management programme and reporting thereon, including

vii.							Monitoring of Impact Management Actions

viii. Monitoring and reporting frequency

vx. Responsible persons

x. Time period for implementing impact management actions

xi. Mechanism for monitoring compliance

| SOURCE ACTIVITY | IMPACTS REQUIRING MONITORING PROGRAMMES | FUNCTIONAL REQUIREMENTS FOR MONITORING | ROLES AND RESPONSIBILITIES (FOR THE EXECUTION OF THE MONITORING PROGRAMMES) | MONITORING AND REPORTING FREQUENCY and TIME PERIODS FOR IMPLEMENTING IMPACT MANAGEMENT ACTIONS |
| --- | --- | --- | --- | --- |
| Mining site/Soil | Possible spillages of petrochemicals. Stripping of topsoil | Checking for spillages on daily basis. Checking correct stripping and stockpiling of topsoil | Manager and Applicant | Daily checking and reporting with Performance Assessment |
| Mining site/ Topography | Concurrent sloping of the sides to 14 degrees | Checking stability of slope and erosion preventive measures | Manager and applicant | Quarterly |
| Mining site/Air quality | Dust pollution from mining activities. | Regular wetting of roads and stockpile area where loading take place. | Manager and applicant | Daily |
| Mining site | Chemical toilet | Make sure that it is used and hygienic. | Manager and Applicant | Weekly. |

- INDICATE THE FREQUENCY OF THE SUBMISSION OF THE PERFORMANCE ASSESSMENT/	 environmental audit report.

Annually

- Environmental Awareness Plan

- Manner in which the applicant intends to inform his or her employees of any environmental risk which may result from their work.

Raubex Construction (Pty) Ltd.will contract DERA Environmental Consultants to inform the employees after the EMP was approved.

The following guidelines will be used:

- Communication

- Urge

- Leadership

- Teamwork

- Understanding

- Recognition

- Empowerment (CULTURE).

- (2)  Manner in which risks will be dealt with in order to avoid pollution or the degradation of the environment.

The risks will be dealt with by proper management actions as described in 1d.

- Specific information required by the Competent Authority

(Among others, confirm that the financial provision will be reviewed annually).

The quantum for rehabilitation liability will be reviewed with the performance assessment on annual basis.

- UNDERTAKING

The EAP herewith confirms

- The correctness of the information provided in the reports;

- The inclusion of comments and inputs from stakeholders and I&APs;

- The inclusion of inputs and recommendations from the specialist reports where relevant; and

- That the information provided by the EAP to interested and affected parties and any responses by the EAP to comments or inputs made by interested and affected parties are correctly reflected herein.

Signature of the environmental assessment practitioner:

DERA Omgewingskonsultante (Pty) Ltd.

Name of company:

1 February 2018

Date:

-END-