IKWEZI MINING 20MVA 88/22kV SUBSTATION AND ASSOCIATED INGAGANE/UTRECHT/IKWEZI 88kV POWERLINE

ENVIRONMENTAL MANAGEMENT PLAN (EMP)

CONTEXTUAL FRAMEWORK

1. EMP Structure

This EMP is structured as follows:

- Background information regarding the proposed project

- Brief description of the EMP

- Purpose of the EMP

- Environmental specifications applicable to the design and planning of the proposed project

- Environmental specifications for the construction phase of the proposed project

- Environmental specifications for the operational phase of the proposed project

- Environmental specifications applicable to the decommissioning of the project

- Relevant environmental procedures are attached

1.1 Needs and desirability

Ikwezi mining (Pty) Ltd has applied for electricity supply from Eskom to provide the required voltage capacity for its mining operations. Eskom identified the nearest and suitable powerline where the electricity can be sourced from. The electricity will be brought to a new substation along a new powerline. The nearest existing powerline for connecting the proposed powerline and substation is 1 Ingagane/Utrecht 88kV which runs north of the study area. The new substation shall step-up the voltage capacity which will provide the needed capacity to the mine. The project will therefore involve the construction of a 5km double powerline from existing 1 Ingagane/Utrecht 88kV powerline to connect to the new Ikwezi Mining 20MVA substation. The powerline structures will be made of steel and will suspend conductors, figure 1. A typical substation consists of a control room, high voltage yard (HV) supporting electrical infrastructure such as transformers, radio tower and other steel electrical structures connecting the in-coming and out-going conductors linked to associated powerlines figure 2. The HV yard is surrounded by palisade fence which acts as safety barrier. Adjacent to the HV yard is an area that is stabilised after being disturbed during construction. The standard dimensions of a substation are 100mx100m.

The substation will also provide electricity to the neighbouring communities. The local network is unable to meet current and forecasted power demands in the area and is thus not able to provide power for new connections. The proposed substation will enable electrification of surrounding communities. The nature of the proposed development triggered the requirement for environmental authorisation in terms of sections 24(5) and 24M of the National Environmental Management Act, 1998. The EIA regulations as promulgated in terms

1.2 Locality

The study area was located during the planning stage of the project. Location is based on the location and availability of electricity network to provide the needed capacity. Planning engineers study the nearest network and determined the most suitable connections points. Sites visits are undertaken by the project team to assess the suitability of the identified connection points. Preliminary environmental investigations are then conducted to assess alternative corridors as well as environmental constraints within the study area and along the alternative corridors. Baseline surveys based on ortho-photo maps of the area and preliminary site visits enabled the demarcation of the study area, figure 3. During planning the suitable connecting points were identified in the Tendeka rural area located within Dannhauser municipality which is part of Amajuba district municipality, northern KwaZulu Natal. The demarcation of the study area was based on the predetermined width of the study area, landform (terrain) and landuse patterns. The width of the study area is approximately 3km and the length is 5km (based on the connecting points)

Properties within the study area are owned by private individuals, Ingonyama trust and Regional and Land Affairs. A couple of visits to the study area were undertaken to assess the area and record findings. The main landuse within the study area is dense housing settlement, subsistence farming, small-scale livestock along open grasslands, and mining located outside and south of the study area. District road P272 cuts through the housing settlement and the study area. The road links Tendeka with the town of Dundee in the south and Osizweni Township in the north. Terrain within the study area is steep along the west of the study area and undulating from the base of the hill rolling over an extended area east of the study area. Tendeka River meanders through the study area with its riparian area prevailed by erosion in some area. In some area erosion has formed extensive riparian channelled wetlands and floodplains along the riparian area.

1.3 Focus of the Environmental Management Plan

The focus of the Environmental Management Plan is to develop an operational management plan for the life cycle of the development.

1.4 Assumptions and limitations

It is assumed that the applicant, Eskom Holdings SOC and the appointed contractor will dutifully ensure that recommendations in the EMP are fully implemented during the construction of the project. Eskom Holdings is responsible for the implementation of recommendations during planning, operation, decommissioning and closure of the project. Adequate funds needed for rehabilitation will be made available at planning stage of the project.

1.5 Limitations

Engineering and soil conservation measures to control the widening of dongas and sheet erosion are not mentioned in this report. The provincial department of agriculture should be contacted for provision of such measures.

1.6 Background to this project

The main aim of the site specific EMP is to provide recommendations on the mitigation of potential negative impacts identified in the basic assessment report of this project and on mitigation of potential impacts during the life-cycle of the project. It considers conditions of each construction site and provides mitigation measures relevant to each site. The preferred powerline corridor 1 is located along west of the study area. The proposed powerline will be located along the preferred corridor and will be connected from 1 Ingagane Utrecht 88kV powerline to the proposed 88/22kV Ikwezi substation. Ikwezi Mining Ltd (Pty) has applied for electricity for their mining operations from Eskom Holdings SOC. The powerline is needed to bring power the mine and the substation will strengthen the capacity supplied to the mine and bring the power closer to the mine. Ludloko Development and independent consulting environmental company was appointed by Eskom Holdings Soc to undertake the basic assessment as promulgated in the National Environmental management Act No 107 of 1998. The EMP is provided as per EIA regulations stipulated in Government gazette No 33306.

1.7 Description of the preferred corridor

The corridor runs along open grasslands which are intercepted by eroded drainage lines which connect to a Tendeka river. The corridor also traverses across floodplains that have turned donga from extensive erosion caused by heavy floods. The area is predominantly used for grazing. Overgrazing has reduced the population species resulting in low biodiversity composition.

1.7.1 Soils

The soils are erodible and are susceptible to stress such as overgrazing and disturbance exposing them to other factors such as wind and rain. Their high clay content makes the soil crack when dry this leads to erosion. Excessive driving and lack of rehabilitation of bare areas will make the soils susceptible to erosion.

1.7.2 River, floodplain and drainage lines

Floods have lowered the river bed to its bedrock. This has extended the floodplains away from their historic level this is caused by the collapsing of the river banks. The extent of the eroding floodplain differs along the length of the river. Along the west of the study area even though eroded the floodplain is narrower while on the east of the study area it is much wider making access across the river difficult.

1.7.3 Grasslands

The grasslands in the area are not considered vulnerable or endangered. They perform an important ecological function of protecting the underlying grass from erosion. Overgrazing is evident by low biodiversity. Compaction of the grass especially at the edge of drainage lines of floodplain by livestock has led to collapse of soil structure at the edge of the eroding drainage line. Disturbance to the grass should be as minimal as possible and disturbed areas should be grassed.

1.8 Legal framework

Section 24 of the Constitution, 108 of 1996:

Everyone has the right-

(a) to an environment that is not harmful to their health or well-being; and
(b) to have the environment protected, for the benefit of present and future generations, through reasonable legislative and other measures that-

(i)  prevent pollution and ecological degradation;
(ii) promote conservation; and
(iii) secure ecologically sustainable development and use of natural resources while promoting justifiable economic and social development

National Environmental Management Act 107 of 1998

The act provides an overall framework for environmental management in South Africa to ensure active protection and responsible utilisation of the environment.

National Water Act No. 36 of 1998 – Section 19

The purpose of the Act is to ensure that the country’s water resources are protected, conserved, managed and controlled in a way that prevents degradation and reduction.

Development Facilitation Act No. 67 of 1995

The act facilitates release of land for development / planning.

KZN Planning and Development Act (PDA) No. 5 of 1998

The act replaces the Town Planning Ordinance. Responsibility for preparing development plans, development control, special case areas, facilitating development through an integrated approach to planning.

Conservation of Agricultural Resources Act, 1983 (Act No 43 of 1983)

The objective the Act is to provide for the conservation of natural agricultural resources of the country by the maintenance of the potential of land, by the combating and prevention of erosion and of the weakening or destruction of the water sources, and by the protection of the vegetation and combating of weeds and invader plants.

Soil Conservation Act no 76 of 1969

Attempts to minimise soil erosion, this act has a particular effect on the estuaries

National Environmental Management: Waste Act (Act 59 of 2008)

The Act provides for:

- Reforming the law regulating waste management in order to protect health and the environment by providing reasonable measures for the prevention of pollution and ecological degradation and for securing ecologically sustainable development

- Specific waste management measures

- The remediation of contaminated land

- Creates a system for listing and licensing of waste management activities

- Waste management activities above certain thresholds are subject to a process of impact assessments and licensing, or a requirement to comply with certain additional standards

National Heritage Resource Act No. 25 of 1999 (NHRA)

- Promotes good management of national estate and to encourage communities to protect their heritage for the benefit of future generations

- The Act considers resources of cultural significance or other special value for the present and future generations.

- The Act considers the demolition of structures older than 60 years and provision of permits to do so, burial grounds and graves, heritage resource management

NEMA: Biodiversity Act No. 10 of 2004

The purpose of the Act is to protect the country’s biodiversity within the framework of NEMA so as to protect species and ecosystems. The Act must read together with NEMA

Fencing Act (Act no 31 of 1963)

The Act enforces the erection of fences along determined borders. It also prohibits of damage to a property owner’s gates and fences.

The Eskom Act 40 of 1987 as amended by the Eskom Amendment Act 51 of 1991; section 11 and 12 authorises Eskom to provide electricity to applying customers

Generation and supply of electricity within the Republic of South Africa is subject to the right of local authorities and holders of licenses under the provisions of the Electricity Act, 1987 investigate, establish, acquire, maintain, co-ordinate, amalgamate and carry on undertakings to provide an efficient and cost-effective supply of electricity to anybody or person in the republic.

National Forest Act No. 84 of 1998

No person may cut, disturb, damage or destroy any indigenous living tree in a natural forest except in form of licence issued under section 7(4) or section 23 or an exemption of the provisions of this subsection published by the minister in the Gazette. Section 12-16 deals with the protected tree species, no person may cut, disturb, damage or destroy or remove any protected tree or collect, remove, transport or export, purchase, sell, donate or in any other manner acquire or dispose of any protected tree except under a licence granted by the minister.

Regional Environmental Planning context

The Amajuba District Strategic Environmental Assessment (SEA) highlighted the erosion risk potential is high in the central regions of the district and low in the North Western and Middle Eastern regions of the district. The district SEA also indicates that the intrinsic biodiversity value of areas within the district of high and intermediate value. In the north, characterised by high lying and mountainous terrain, there is a large % of the land covered by important landscapes. This refers to the existence of important areas such as wetlands, grasslands, forests, vegetation communities of significant importance. Included in these areas are high value landscapes, around Charlestown and Amajuba Forest, Zaaihoek Dam and the Inkosi Shabalala Land reform project. These high value areas are also known those where Oribi habitat exists in the Chelmsford area and while some of it is conserved in the Chelmsford nature reserve KZN wildlife.  Wattled Crane nesting sites also occur in the district, Rudd’s Lark and medicinal plant species are found. These habitats lie outside the study area.

The study area is characterised by areas of ‘intermediate’ biodiversity value which are predominantly those areas where there are communities of important vegetation species (particularly grassland species), wetland and riverine environments and bird life. The study area is characterised by eroding wetlands, eroding and wide riparian areas, dongas along drainage lines, eroding river banks and lowered river bed.

Of concern are the extensive areas of donga and sheet erosion – widely spread throughout the district due to the loss of valuable topsoil. As mentioned above the study area in some areas is incised by donga and sheet erosion. The recommendations from the SEA include:

This issue could be mitigated by the:

1. Removal of alien invasive plants, and replacement by fast growing indigenous plants - job creation;

2. Rehabilitation of degraded / quarried areas with indigenous plants and innovative engineering methods;

3. Prevention of overgrazing / over-utilization;

4. Protection and rehabilitation of soil resources (to support the range of agricultural activities in the area, and consequently food resources).

5. Environmental awareness could be extended to farmers of the region. There is a large database of land quality and soil type for the different bioregions. If the farmer where made aware of the information and was thus able to access it for his area this would unable the farmers to identify the potential for growing different crops. Identify more sustainable farming practices and thus possibly improve yield.

2. Purpose of the EMP

The main objective of the EMP document is to provide guidance on the development and information required to implement effective management and monitoring measures for the listed priority impacts identified in the assessment process.

2.1 Objectives of the EMP

- To identify activities /that are pertinent to the project cycle

- To understand associated impacts and causes

- To ensure that the risks are known and managed

- Provide effective and measurable mitigation measures

- Assign appropriate responsibilities

- Allocate reasonable timeframes

- Quantify target the operation wishes to achieve through mitigation measures

- Provide Key Performance Indicators (KPI) in order to monitor performance on extent to which targets are achieved

2.3 Non-compliance with the EMP

Outlined below are a number of steps, relating to increasing severity of environmental problems, which will be implemented.

Step 1

The ECO discusses the problem with the contractor and/or non-complying party, and all work out a solution together. The ECO records the discussion and the solution implemented.

Step 2

A serious infringement occurs and the ECO either observes it or is made aware of it. ECO notifies the non-complying party in writing, with a deadline by which the problem must be rectified. All costs will be borne by the contractor.

Step 3

The ECO shall order the contractor to suspend part, or all, the works. The suspension will be enforced until such time as the offending party(ies), procedure or equipment is corrected and/or remedial measures put in place if required. No extension of time will be granted for such delays and all cost will be borne by the contractor.

Step 4

A breach of contract may constitute the removal of a contractor and/or equipment from the park and/or the termination of the contract, whether a construction contract or an employment contract. Such measures will not replace any legal proceedings that Eskom may institute against the contractor

3. ROLES AND RESPONSIBILITIES IN THE IMPLEMENTATION OF THE EMP

3.1 Environmental Assessment Practitioner (EAP)

The EAP Duduzile Ngidi is registered with South African Council for Natural Scientific Professions (SACNASP) as a Professional Natural Scientist, registration number 400033/01 in terms of section 20(3)(a) of the Natural Scientific Profession Act, 2003. The EAP has over 10 years experience in the EIA field. The EAP has the following experience:

- Good theoretical training and practical experience on undertaking Environmental Impact Assessment (EIA) study, execution of the EIA plan of study and EIA project management, this include:

- Knowledge of environmental legislation and regulatory requirements and their incorporation into the EIA process

- Understand and able to present the business case for the EIA projects to stakeholders and able to incorporate the business case in the EIA report

- Good understanding of impact assessment methods, from simple checklist to complex calculations

- Good command of analysing technical information and communicating it to project team and stakeholders

- Sound knowledge of the compilation of Environmental Management Plans/programmes (EMP) report based on the findings of the EIA study, including synthesis and incorporation of recommendations from specialist studies

3.2 People involved in the construction, rehabilitation activities and application of mitigation measures are:

- Project manager and coordinator (PM; PC)

  - Project management and project time frames

- Construction supervisor (CS)

  - Construction and safety monitoring

- Resident environmental control officer

  - Appointed to monitor and record compliance and non-compliance to the ROD conditions and to other relevant legislation and application of the EMP

- Eskom environmental officer (EEO)

  - To monitor compliance

- Environmental consultant

  - Compile the EMP

  - Distribute EMP to authority, contractor and project engineer

- Community liaison officer

  - To liaise with the community

  - To note and keep a record of community concerns

- Environmental Control Officer (ECO)

  - Monitor construction on environmental performance and compliance monthly and when applicable

  - Report on non-compliance

  - Audit construction activities

  - Record findings

3.3 Appointment of an Environmental Control Officer

An Environmental Control Officer has been created to ensure that the mitigation measures and other requirements set forth in the EMP are adhered to. It is recommended that Eskom appoint an independent Environmental Control Officer (ECO) during the construction phase of the project.

3.3.1 The following guidelines apply to the functions of an ECO:

- The ECO should be conversant and has full understanding of the contents of the Environmental Management Plan

- The ECO should and explain the EMP to the contractor, the site staff, the supervisors and any other relevant personnel or I&AP’s.

- The ECO would have to be on site on a regular basis – preferably once a month to supervise environmental actions associated with construction activities.

- The ECO should be able to understand, interpret, monitor, audit and implement the EMP.

- The ECO must then give feedback of the audits to Eskom and Contractors. This must be in the form of a written report .

- The ECO must ensure that the contractor understands what is to be done to rectify and address any problems that have arisen from the audit.

- Be familiar with environmental and other related legislation including contraventions and penalties

- Attendance at scheduled project meetings

- Monitor the contractor’s compliance with the EMP on site

- Conduct monthly observation and environmental audits of all work areas

- Check of all completed work (storage of topsoil, its application, grassing etc)

- Keep all records of project monitoring for audit

- To be responsible for all environmental communication on the project with the project team

- To discuss queries from I&AP

- To channel/direct all public statement on environmental issues to Environmental practitioner

- Discuss content of the EMP with construction supervisor and with Project management

- Monitor compliance to ROD and EMP recommendations

- Attend to environmental incidents and liaise with construction supervisor to implement prevention measures

- Keep a regular photographic record of all environmental incidents

- Report environmental incidents to Eskom Environmental Senior Advisor and to Environmental Practitioner assigned to the project

- Liaise with Drizit (Oil and Chemical Management Specialists) on correct management procedures. Contacts provided in section 12 end of document

- Attend on-site meetings

- Bring to the attention of project team environmental related problems and report on progress

- Check quality of rehabilitation work

- Discuss poor quality with construction supervisor and ensure proper implementation of procedures

- To provide environmental practitioner with progress reports

- Approve completion of each rehabilitation work

3.4 Tasks for Project Co-ordinator/Manager

The project manager will be responsible for ensuring that the EMP and associated documents are compiled with the contractor.

- Must be familiar with the content of the EMP

- Discuss the EMP report with the project team

- Liaise with Eskom Environmental practitioner

- Notify landowners and local municipality about project schedule and activities prior to construction

- Notify landowners prior to entering private property

- Assist with the appointment of Environmental Control Officer (ECO) and Community Liaison Officer (CLO)

- Monitor progress and environmental issues associated with the project

- Ensure that all environmental recommendations are implemented and adhered to

- Ensure that there is adequate finance for environmental incidents that need to be corrected

- Record and report environmental incidents to Eskom Senior Environmental Advisor

- On completion of the project, ensure that the community and other stakeholders are satisfied with the state of their area

- Become involved in the EMP process and make necessary changes or modifications to the EMP

- Ensure that damages to property and land are addressed to the satisfaction of the landowner

- Ensure that all damage and servitude acquisition compensations are paid

- Chair any environmental incident and environmental non-conformance enquiries

3.5 Construction Supervisor Tasks

- Implement the recommendations of the EMP document

- Ensure adherence to safety standards and procedures as stipulated in the Contractor Project Quality Assurance file containing Safety, Health and Environment (SHE) standards

- Ensure induction material includes project appropriate environmental issues

- Discuss appointment of ECO and CLO with the project manager/coordinator

- To liaise with ECO on compliance and completion of rehabilitation tasks

- Purchase all necessary and correct rehabilitation material; to be delivered on-site in time

- Monitor the application of rehabilitation specifications for powerline and substation construction such as

  - Grassing of disturbed and denuded areas (along the powerline, substation, vehicle tracks and campsite)

  - Cutting of identified trees along the powerline servitude

  - Application of herbicides to stumps

  - Construction of access roads

  - Construction of drains and dissipaters around the substation

  - Creation of a temporary cement slurry holding pan

  - Disposal and removal of construction waste

- To have knowledge of the contents of the “Procedure for vegetation clearance and maintenance within overhead powerline servitudes and on Eskom owned land” document. The contractor and the ECO are to have knowledge of the content of the document and are to discuss the content with the construction staff.

- Maintain site documentation related to environmental management (permits, EMP methods, ROD, audit reports, waste removal etc) Documentation to ne maintained on the relevant site Document Control System

- Provide emergency procedure for environmental incidents

3.6 Eskom Environmental Practitioner

- To assist in dealing with environmental incidents

- To ensure that quality work has been done

Table 1 Summary of environmental impacts

| Activity | Effects | Mitigation |
| --- | --- | --- |
| No building of powerline and substation | There will no power to the mine and no job creation The area will not improve on its revenue and meeting employment targets Electrification backlogs will not be addressed | Provision of electricity outweighs the identified negative impacts Existing erosion will not be enhanced by the location of the powerline however, this provides the opportunity for regional department of agriculture to be involved in providing solutions to the gullies and sheet erosion |
| Locating powerline along corridor 1 | Corridor 1 even though has environmental constraints such as sheet erosion and gullies, the powerline will not impact on the sensitive areas Corridor 1 is has a flatter terrain, is accessible, is outside residential area Has narrow eroding gullies which the powerline can span across | Towers are to be located outside wetlands, riparian area and drainage lines This will prevent damage to the areas |
| Locating powerline along corridor 2 and 3 | Both corridors have houses built at randomly restricting location of a powerline due to servitude constraints and close proximity to houses | Avoid locating a powerline along corridor 2 and 3 |
| Locating powerline along corridor 4 | Even though corridor 4 has open terrain the start of the corridor has an extensive degraded riparian area It will not be possible to span across it Towards the end of the corridor houses exist and are built randomly causing servitude constraints The corridor is also longer | Avoid locating a powerline along corridor 4 |
| Poor environmental planning | Bad planning can place the project along sensitive environments even through houses | Planning should identify the sensitivity of the area and identify less sensitive areas for locating powerline Planning should involve tower designs and location |
| Poor structural designs | The area is eroding and requires stable and durable pile foundations Geo-tech studies undertaken during dry weather season showed that dug holes did not collapse Locating powerline along corridor 1 will have longer spans between towers to locate them across riparian areas therefore structures suitable for spans are required | Designs need to consider constructing the powerline during dry weather season |
| Offering manual labour | Provides temporary jobs | Use of manual labour to be discussed with the local authorities, chiefs, Indunas and councillors |
| Establishing office and campsite along sensitive areas | Locating office site within or close to sensitive habitats such as wetlands, drainage lines, pans, or the riparian area | Office and camp sites are to be located in relatively flat area away from sensitive habitats Use of the land for locating the office and campsite needs to e discussed with the landowner All safety precautions must be adhered to Site must have a perimeter fence to contain all the activities and limit disturbance and provide safety to project material Contractor to provide site plan |
| Poor environmental assessment process | Environmental issues will not be identified and mitigated and this will lead to further damage of the environment | Identification of issues must be discussed with the project team |
| Poor identification of project impacts and constraints | Environmental constraints will pose safety problems to the project infrastructure | Mitigation measures must be fully implemented |
| Excessive driving along grasslands | This will lead to soil compaction, removal of vegetation, erosion and later gully erosion Affects the filtration of water down the soil profile leading to breaking of the soil and to erosion | Driving is to be done along existing tracks No multiple tracks are to be created |
| Site clearance | Removal of ground cover that protects underlying soil and loss of soil | Removal of vegetation must be limited within demarcated work area To minimise impacts of ground disturbance construction must be done outside rainy season |
| Poor storage of topsoil | Mixing of topsoil with subsoil and/or construction rubble will affect its capacity to be used for grassing of disturbed areas | Topsoil must be stored separately to sub-soil and construction rubble and away from vehicle movement |
| Potential erosion of bare areas | Loss of ground cover leads to loss of habitat and landuse Loss of soil lead to irreversible damage to the environment | Bare areas must not be exposed for long periods Rehabilitation must be done as soon as possible |
| Driving along sensitive areas such as wetlands and riparian areas | Damaging sensitive ecosystems Impacting on the water system Increasing effects of floods as a result of dysfunctional wetlands and riparian areas | No driving through wetlands and riparian areas including through the river must not be done These areas need to be demarcated in order to avoid driving or interfering with them |
| Affecting drainage lines | Will impact on channelling of run-off along the ground |  |
| Location of towers within sensitive ecological systems | Disturbance to these ecological systems renders them dysfunctional affecting interlinked systems and specialised habitats for various species They also affect the ecological roles for these systems leading to degradation and pollution of the water systems | Towers are to be located outside these ecological systems Towers are to be located along the outer edge of the riparian area and beyond 32m of the river bank edge |
| Driving along riverbanks | Damaging fragile river banks that are already eroding on some parts of the river | No driving along river banks or within 32m buffer area from the edge of the river bank |
| Widening of gullies | As a result of driving along edges of eroding gullies Cutting open gully walls This caused extensive erosion into the river leading to pollution Could eventually reach tower foundations and threaten the stability of the powerline infrastructure | Involvement of the local department of agriculture to address the erosion in the area is imperative. The local department of soil conservation has the tools and resources to address the erosion problem Communities can be involved by the department to be made aware of the causes of over-grazing |
| Poor waste management | Scattered litter around the site | All litter around the work area is to be removed and disposed at registered landfill sites |
| Dust | Dust will be blown towards the adjacent neighbouring homesteads | Araes that could generate dust need to be watered |
| Lack or poor rehabilitation of denuded areas | Loss of vegetation and subsequently loss of soil | Rehabilitation plan must be provided Indigenous material for rehabilitating must be used |

4. PRE-CONSTRUCTION MANAGEMENT PLAN – PLANNING, DESIGN AND CONSULTATION STAGE

4.1 Pre-construction stage involves:

List of associated impacts at this stage are based on data sourced from strategic environmental assessment or from other existing high level data such as data from ortho-photos and literature.

Objective

To minimise potential environmental risks at early stages of project cycle (planning and design) stage

Activity/Aspect

- Lack of environmental input in planning and design

- Design that does not fit into the site footprint

- Poor infrastructural design

List of associated impacts

- Lack of proper communication with stakeholders

- Poor selection of corridor and substation site

- Negative impacts on people

- Affect landuse

- Impact on cultural and landscape character

- Site footprint extending on private property

- Affect property borders

- Intrusion on private property

Possible Causes

- Lack of proper planning

- Location on sensitive environments

- Unstable structures as a result of poor design

- Relocation of people

- Location on important landuse and cultural sites

- Lack of proper consultation

Risks

- Non-compliance and fines

- Disruption of interconnected ecosystems and sensitive habitats

- Emotional stress on affected people

- Irreversible destruction of landscape and cultural sites

- Compromising safety standards

- Disruption on electricity supply as a result of poorly constructed structures

Mitigation

- Identify environmentally suitable and stable substation site and powerline corridor

- Ensure that appropriate authorisation of design structures is done

- Liaise with stakeholders and interested and affected parties to ensure that the location is acceptable

- Location of access road within the defined servitude

- Incorporating geotechnical recommendations that will minimise impact on receiving surface and sub-terrain land and erosion control

- Finalise the location of the site in relation to associated infrastructure components such as incoming and outgoing lines

- Incorporate recommendations from strategic environmental assessment (SEA)

- Planning and design must fit the receiving environment and ensure minimal disturbance

- Planning location of project site, campsite, vehicle parking bay, office site

Targets

- Avoidance of sensitive environmental sites

- Proof of comprehensive public participation

- Approval of project set-up (plan and design) internally and externally

- No change in site footprint and access road servitude once established unless approved by affected landowner(s)

- Evidence of consideration of geotechnical information where applicable

Responsible

Project Engineer, Project Surveyor and Environmentalist

Activity/Aspect

Lack of identification of sensitive habitats

This happens when specialist studies had not be undertaken. This could lead to disturbance to the integrity of the habitat and impacting on ecological functions of the habitat. Adjacent habitats are ecologically integrated by having associated functions. Disturbing one habitat could lead to interruption of the integrated ecosystem

List of associated impacts

- Degradation and destruction of sensitive habitats

- Transgression of NEMBA

- Locating of towers along water logged habitats such as wetlands

Possible causes

- Lack of using specialists

- Lack of demarcation of the sensitive habitats

- Lack of implementation of mitigation measures

Risks

- Litigation

- Irreparable destruction of sensitive habitats

Mitigation

- Biodiversity and other specialist to undertake specialist studies

- Implementation of recommendations from specialist studies

- Monitoring of construction along these habitats

- Ensuring that the habitats are not disturbed

Targets

Identification and demarcation of sensitive habitats

Responsibility

- ECO, Project coordinator, Eskom officer

Activity/Aspect

Poor identification of project impacts and constraints

This will lead to further exacerbation of impacts and irreversible damage

Objective

To identify all pertinent and associated impacts in order to minimise the level of risk

Possible causes

- Lack of professional experience

- Lack of communication with specialists and other stakeholders as well as to the landowners

- Poor investigation skills

Risks

- Non compliance to related legislation

- Litigation

- Conflicts with stakeholders and landowners

- Further degradation of the environment

- Impacting on biodiversity

- Impact on the powerline

Mitigation

- Quality assurance check by other professionals

- Liaisons with Ezemvelo KZN on conservation issues within the study area

- Proper liaisons with municipality on spatial planning

- Proper discussion on identified impacts

Targets

- Identification of all impacts

- Recording of all identified impacts

- Liaisons with all relevant stakeholders to ensure extended input about the study area

Responsibility

- ECO, project team, project coordinator

Activity/Aspect

Poor identification of environmental constraints

Objectives

To identify constraints posed by the environment on the project and how to mitigate against that

Possible causes

- Poor identification of the environmental constraints

- Inability to identify the constraints

Risks

- Locating powerline along problematic areas

- Making accessibility to the powerline in the long term difficult

- Non-compliance to relevant legislation

- Litigation

Mitigation

- Ensure identification of environmental constraints

- Discuss the environmental constraints with other professionals such as Eskom environmental officers

- Discuss the findings with the project engineer and project team members to formulate a solution with regards to design and tower type

- Full application of recommendations from all relevant parties

- Poor undertaking of sufficient assessment to identify the ecological issues along each of the proposed corridors, and the significance of the likely impacts of a proposed power line located within those wider corridors

Targets

- Ensure that powerline design is suitable for the environment

- Minimal disturbance to the environment

Responsibility

- ECO, specialists, Eskom officer

Activity/Aspect

Poor selection of suitable corridor and substation site

This could lead to further disturbance of the receiving environment and threats to infrastructure stability

Objective

To select corridor and substation site approved by all stakeholders and affected landowners

Possible causes

- Lack of professional experience

- Lack of communication with specialists and other stakeholders as well as to the landowners

- Poor investigation skills

Risks

- Non compliance to related legislation

- Litigations

- Conflicts with landowners and stakeholders

- Bad corporate reputation

Mitigation

- Ensure proper liaisons with specialists

- Discuss the alternative corridors and substation sites with other professionals

- Ensure that recommendations from specialists have been considered

- Ensure receiving comments from all relevant parties

- Ensure incorporation of their recommendations

- Ensure that all relevant issues have been covered

Targets

- Proper consultations with stakeholders, landowners and other professionals

- Ensure that the suitable corridor and substation site have been approved by all relevant parties

Responsibility

- ECO, Eskom officer

GENERAL IMPACTS (Applying to both powerline and substation)

Activity/Aspect

Establishing campsite and office site

Location and establishing of campsite should be preferable along flatter areas away from sensitive areas such as riparian areas, wetlands and steep areas

Objectives

- To miminise impacts associated with the establishment and operation of construction and camp sites

- To minimise and control damage to receiving environment and to entrench accepted behaviour amongst communities construction

Potential impacts

- Loss of ground cover due to clearing

- Creation of a temporary access road

- Uncontrolled movement of run-off from temporary water tap

- Oil leaks from standing vehicles

- Oil leaks from serviced vehicles on site

- Waste pollution

- Unsafe storage of construction material

- Uncontrollable management of chemical leaks due to improper storage

- Pollution of nearby streams or drainage lines

- Illegal use of land for locating campsite or vehicle parking bay

- Improper behaviour by construction staff with residents

- Spread of fire from the site or through the site due to lack of fire belts

Possible Causes

- Lack of proper planning at planning and design stage

- Soil erosion

- Creation of water gullies from uncontrolled run-off

- Lack of discipline to collect waste daily

- Lack of planning for campsite, vehicle parking site, office site and storage sites

- Lack of placing impermeable moulds to contain vehicle oil leaks

- Uncontrolled storage of construction material

- Lack of education on good behaviour supported by contractor worker policy and health issues

Risks

- Water and soil pollution

- Permanent loss of topsoil

- Waste carried by winds and polluting neighbouring properties

- Explosion of hazardous material and inability to control the explosion due to inaccessibility as a result of being covered by other heavy construction material

- All material and equipment storage areas (including storage areas for hazardous substances such as fuel and chemicals) should be marked.

- Allocation and location of waste bins

- Litigation and conflicts as a result of using land illegally

- Health problems in the community caused by improper behaviour from construction staff

- Spread of human transmittable diseases

- Loss of property and assets due to run-away fire

- Bad reputation on Eskom and contractor

- Loss of business by contractor

- Non-compliance to municipality by-laws such as creation of fire belts

Mitigation

- Project engineer provide a plan showing the site plan

- The plan must indicate access roads to be used, construction lay-down

- Proper consultation for use of new land for establishing a camp site

- No removal of ground cover

- Firebreaks should be established around the construction site, camp site (where applicable), around vehicle parking bays (where applicable) and around office site (if outside construction site)

- A fire-break shall be cleared and maintained around the perimeter of the site including office site

- All conditions incorporated in the requirements of the Occupational Health and safety Act shall also be implemented

- Camp site, office site, and construction site must be fenced off to clearly demarcate the sites and contain all activities within the sites

- Notices must be posted on the fences and gate of the construction site to make the members of the public aware of activities and safety issues

- Upon completion of construction work the camp site must be fully rehabilitated by removing the temporary surface, ripping the area to loosen the soil and rehabilitating according to the landscape development plan for the project

- The Contractor shall provide adequate facilities for construction staff so that they are not encouraged to use natural resources from surrounding natural environment to supplement their needs

- Burning of wastes and clearing of vegetation is prohibited

- Fire fighting equipment and material shall be supplied at all sites and located at accessible areas

Targets

- No use of new land without authorisation

- Proper and effective plan showing planning for location of campsite, vehicle parking bay and office camp

- Zero waste on site, campsite, office site and neighbouring properties

- No sign of oil pollution on soil

- No complaints from community about misbehaviour of construction staff

Responsible

Project manager, ECO and EEO

Activity/Aspect

Provision of temporary employment and skills transfer where possible

Temporary employment can offer part-time job opportunity and also improve on the work skills

Objective

To provide temporary employment to unemployed locals

List of associated impacts

- Benefits to local community

- Disappointment with disregarding of use of manual labour

Possible causes

- High unemployment

- Desperation in need of any temporary work

- Strongly expected by community

Risks

- Bad corporate image

- Using people outside area for manual work

- Does not comply with national development plan and poverty alleviation strategies

Mitigation

- Liaise with local authorities for recruiting manual labour

- Use labour expert to recruit, register explain legal aspect of the job

- Ensure safety standards and requirements are explained

- Limit access into the construction site only to the employed and trained construction staff

Targets

- Liaise with local authorities to discuss manual labour for the project

- Agreed upon number of temporary employment

- Agreed upon number on skills transfer

Responsible

- Contractor, local municipality and ward councillor

Activity/Aspect

Waste management

Poor waste management will result in the pollution of both land and water

Objectives

To prevent pollution from waste

Potential impacts

- Ground and water pollution

- Litter spreading to adjacent areas

- Unsightly litter along the project area

- Lack of waste collection bins

Possible Causes

- Indiscriminate dumping

- Lack of awareness to project staff about waste management

Risks

- Mon-compliance

- Liability

- Bad company reputation

- Waste becoming nuisance

Mitigation

- A waste management plan must be presented and discussed

- Waste must be separated into different categories

- Recyclable material must be taken to local recycle centres found at garages, malls etc

- Waste bins must be provided and their use enforced

- All general waste must be disposed at registered landfill sites

- Hazardous waste must be removed by qualified company to authorised waste treatment centres

- A waste register must be kept on site and provided upon request

Targets

- All waste must be continually removed from site

- Records of recycled material

Responsible

- Contractor, Project coordinator

Activity/Aspect

Rehabilitation of degraded areas (dongas)

In this case to effectively stabilise or halt expansion of existing gullies, practical vegetation and engineering solutions are needed. The local department of agriculture soil conservation section does offer such solutions and should be contacted

Objectives

- To miminise expansion of gullies and effects of sheet erosion

Potential impacts

- Undermine structural integrity and safety

- The area becomes inaccessible

- Threat to supply of power to the mine and surrounding communities

Possible Causes

- Lack of commitment from the applicant to address the erosion along the preferred corridor

- Delaying involvement of the department of agriculture to combat widening of dongas

Risks

- Structural damage

- Electrocution incidents

- Loss of power to the mine  and surrounding communities

Mitigation

- Map current areas that are degrading and have dongas

- Liaise with department of agriculture (soil conservation) and adjacent landowners to find solutions to address degradation

- Ensure implementation of recommendations from the department for addressing widening of dongas

- Monitor progress on the mitigation measures

- Encourage the department and adjacent communities to protect measures applied to combat widening and cause of dongas and effects of sheet erosion

Targets

- Liaising with the local department of agriculture conservation office

- Implementation of conservation remedies to combat expanding dongas

- Placing of towers outside the donga areas

Responsible

- Contractor, ECO, project coordinator

Activity/Aspect

River and drainage lines

River systems and associated and adjacent habitats are vulnerable to disturbance and as such need to be protected. Driving along or across these habitats or digging them up can destroy them irreparably.

Objectives

To prevent damages to the river system

Possible Causes

- Damaging of river banks through driving or cutting

- Damage of riparian area through driving

Risks

- Non-compliance

- Litigation

- Bad company reputation

Mitigation

- River crossing areas by heavy vehicles is prohibited as access is available

- No driving or modification of drainage lines

- No driving along river banks

- Parking along river bank is prohibited

- The use of rivers and streams for washing of clothes is not permitted

- No washing in the river by staff is permitted

- Use of river water for watering grass requires permit from Department of Water Affairs

- No swimming in the river

- No litter or rubble should be found in the river

- Fishing is strictly prohibited

- Office site is to be located away from river course and drainage lines to prevent draining of chemical spills into the river

Targets

- No crossing of river by heavy construction trucks

- No driving along the river bed

- No use of water for anything

Responsible

- Contractor, ECO

Activity/Aspect

Heritage artefacts

Work must be stopped around the site where archaeological artefacts have been exhumed. Findings are to be reported to project coordinator, ECO and to Eskom

Objective

To sensitise construction staff on heritage artefacts including embedded objects or human bones on construction site

Activity

Encountering heritage artefacts

Potential impacts

- Discovery of valuable heritage artefacts

- Good publicity for Eskom for contribution to heritage protection and reporting

- Damage and destruction of heritage assets

- Non compliance to heritage act

Possible causes

- Lack of knowledge about heritage artefacts by construction staff

- Lack of observation by construction supervisor

- Accidental damage during excavation

Risks

- Fines and litigation

- Bad publicity

- Loss of future work from Eskom by contractor for continued damage to artefacts

Mitigation

- The Contractor shall arrange for Environmental and Heritage Awareness Training programmes for the personnel on site

- Amafa AkwaZulu-Natali should be contacted if any graves are identified during earthmoving activities and all development should cease until further notice

- Amafa AkwaZulu-Natali should be contacted if any heritage objects are identified during earthmoving activities and all development should cease until further notice.

- If an artefact on site is uncovered work in the immediate site should be stopped immediately and the ECO shall be notified as soon as possible

- Contractor shall take reasonable precaution to prevent any person from removing or damaging any such article.

- Work shall resume once clearance is given in writing by an appointed archaeologist.

Targets

- Quick recognition of archaeological artefacts

- Minimal damage to archaeological artefacts

- Reporting on encounters

- Reporting on corrective actions taken

Responsible

Project manager, Project supervisor, ECO EEO

Activity/Aspect

Excessive driving along grasslands

The underground soil within the study area is susceptible to erosion due it is expanding and contracting nature as well as high clay content. Excessive driving across grasslands will lead to erosion. Driving is to be limited to existing tracks. Creation of multiple tracks is prohibited

Objective

To minimise driving along grasslands

Possible causes

Excessive driving

Increased compaction

Potential impacts

Vehicle traffic can destroy vegetation through trampling and can increase soil compaction

Accelerate the establishment of invasive exotic species

Reduce plant regeneration

Reduce root penetration

Physically damage ground cover and underlying soils

Reduce porosity

Decrease filtration of run-off

Increased speed of surface run-off

Form artificial water channels along compacted vehicle paths

Initiate erosion

Risks

Reduced regeneration

Loss of ground cover

Erosion

Gully formation

Mitigation

- Reduce driving along a single path

- After construction loosen soil

- Rehabilitate with suitable grass seed mixture

- Plant on the onset of rainy season

- Prevent disturbance of newly rehabilitated areas

Targets

- Minimal disturbance to ground cover

Responsible

Project manager, ECO

Activity/Aspect

Rehabilitation of bare areas

Construction involves stripping of ground cover thus exposing the underlying soil to erosion impacts. Rehabilitation will prevent potential erosion

Objective

To prevent potential erosion

Activity

To cover areas made bare by clearance of ground

Potential impacts

- Potential erosion

- Loss of soil

- Decline in land potential

- Formation of gullies

Possible causes

- Lack of adequate funds to rehabilitate

- Rehabilitating during dry and cold season

- Poor vegetation used to cover bare areas

Risks

- Erosion

- Threats to powerline and substation structural safety

Mitigation

- Funds are to be made available for rehabilitation

- All bare areas must be identified and measured in order to acquire adequate grassing material

- Grassing plan must be discussed with the project ECO and coordinator

- Good rehabilitation material and suitable material must be used to grass bare area

- Grassing should be done during rainy season

- Perimeter fence must be put around the substation to prevent livestock trampling on newly planted areas

Targets

- Well grassed bare areas

- All identified bare areas must be grassed

- Progress on the growth of grass should be monitored

Responsible

- ECO, project coordinator

5. CONSTRUCTION MANAGEMENT PLAN

5.1 Construction management overall objectives

- To ensure that mitigation measures for all identified potential impacts are provided

- To ensure that mitigation measures are applicable and effective against identified impacts

- To ensure that project environmental awareness is communicated to the involved staff

- To ensure that site specific measures are applied

- To ensure that disturbed areas are rehabilitated accordingly

- Where damage occurs outside the site, that the damage is recorded an d compensations accordingly

- To ensure that emergency preparedness is in place in order to contain consequences of the emergency situation

5.2 Construction involves the following activities:

- Identification of tower positions

- Identifying access roads and tracks

- Establishing camp site (including office, workshop and staff site)

- Demarcation of camp site

- Site plan (including storage areas, office areas, waste collection site)

- Clearing ground for campsite

- Stockpiling  top soil

- Off-loading construction material to camp site and work areas

- Establishing tower positions

- Demarcation of tower work areas

- Demarcation of substation site work area

- Removal of ground cover

- Stockpiling of topsoil and sub-soil

- Archaeological inspection of tower holes

- Mixing of cement

- Erection of towers

- Removal of waste inside the holes

- Pouring of cement

- Application of safety requirements around un-completed work areas

- Rehabilitation of areas around the hole

- Stringing of the conductor

- Erection of substation

- Stabilising and rehabilitation of bare areas around the substation

- Removal of waste

- Dismantling of camp site

5.3 Construction period

- Construction is recommended to be done during the dry seasons

- This will minimise impact on grass and weak underlying soils

- The soils in the area are sensitive to stress, this is made evident by the weathering soil adjacent to cattle paths

- Area gets floods which can make working near the riparian area difficult and unsafe

Activity/Aspect

Construction of powerline

Construction at the start of the powerline to the middle of the study area along corridor 1

- Construction starts along an area free of erosion

- Towards the middle of the corridor erosion occurs

- The erosion starts at the base of the ridge and expands towards the valley and it recedes before reaching residential area

- Towers need to span over the scoured area

Fig 1 showing eroding riparian area

Construction from middle of the corridor to Tendeka River along corridor 1

- The area has a gentle slope towards the river and is characterised by presence of un-channelled and channelled wetlands

- Wetlands appear as depressions and can be recognised by the type of vegetation and wet soils

- These wetlands have water during rainy season or when water table is high

- The corridor is situated towards the edge of the study area thus enables to avoid un-channelled and channelled wetlands

- The crossing area at the river area is narrower, see figure 2, and the powerline will be able to span across the degraded riparian area

- Spanning across the riparian area will lend the tower on a upper terrace/edge of the riparian area

Fig 2 showing a narrow river channel

Construction from Tendeka River to the substation site along corridor 1

- The area is undulating

- Wetlands exist on the east of the corridor

- Gullies flourish along the riparian area

- The corridor runs along elevated ground

- There is a vehicle crossing area with underlying sandstone rock

- The river crossing is possible during dry weather

- The edges of the river crossing may become damaged if crossing occurs during rainy days

- Another crossing is over the bridge along P272

Impact on wetlands

Wetlands systems are considered sensitive therefore no construction is to take place near or within them. Wetlands are protected by law and their protection is important. Due to the high rainfall and undulating grassland plains with pan depressions numerous wetlands can be found in the area. Wetlands also occur along corridor 1 and 4. However, placing of towers will be outside the wetland 32m buffer zone. The main wetland types impacted upon are the floodplains of the small rivers and streams encountered in the study area. No visible, large or pristine wetland pans or other typical wetland types were observed.

Mitigation

- Towers are to be located 32m from the edge of the wetland

- No campsite, temporary storage facility, or any other facility to be erected within 500m of a wetland

- Driving across wetlands is prohibited

Impact on riparian areas and river banks

Riparian areas are vegetated areas adjacent to river banks. When continuously under water they are considered as wetlands. Riparian areas and river banks area also considered sensitive and require protection in order to protect their important ecological function. No towers are to be located within riparian areas.

Mitigation

- Towers are to be located 32m from the edge of the riparian area

- No tower is to be located along a river bank as that would threaten the safety of the tower due to instability of river banks and effects of floods along riparian area and river banks

- No campsite, temporary storage facility, or any other facility to be erected within 500m of a wetland

Drainage lines and river

Drainage lines are part of the watercourse system. They channel seepage and run-off into the local river.  Due to their nature drainage lines and rivers are sensitive and need to be avoided where possible. Corridor 1 and 4 traverse some of the drainage lines. During high rainfall, soils, grassland vegetation and a gradual slope, many of the drainage areas carry water. This is caused by the continual seepage of groundwater in the depressions and the slow run-off of the water. Some of the drainage lines have been eroded and have become dongas. Towers should be located outside the drainage lines and outside the river buffer zone.

Mitigation

- No temporary or other construction facilities to be erected or stored within 100m of the banks of the rivers, or main drainage lines.

- Placing of towers need to be a minimum of 32m from the edge of the river banks or outside of the 1 in 100 year floodline

- Positioning of the concrete foundation for the towers must be a minimum of 32m away from the edge of all drainage lines

- No chemical ablution facilities to be placed within 200m of the banks of any of the rivers, streams or drainage lines

- Portable ablution facilities only to be serviced by registered companies and on a regular basis

- No effluent or sewage to be dumped in the open veld.

- Potable water facilities need to be provided and maintained for construction workers River water cannot be used for construction of towers such as mixing cement

- Premixed cement must be used to establish tower foundations

Crossing of dongas

- Crossing of dongas and existing eroded areas shall be thoroughly planned prior to the start of construction and movement of construction and delivery vehicles

- No opening or driving across dongas should take place as this would lead to further erosion

- No unnecessary clearing of grass cover beyond the tower bases which will expose the soil which is prone to erosion

- Driving to limited to exist tracks or new tracks so as minimise damage to grass cover

- No indiscriminate driving should take place

- Exposed ground along the newly made vehicle tracks should be rehabilitated

- All cleared areas must be ripped and rehabilitated after construction. The top 30mm layer of topsoil must be neatly removed and stockpiled in heaps

- The stored topsoil must not be mixed with subsoil and construction spoil as it will be used for grassing

Activity/Aspect

Digging of holes for pile foundations

Digging of holes is to be strictly limited to specified hole dimensions. Work area is to be demarcated with a safety tape and all disturbances are to be contained within it. This will limit extent of disturbance.

Objective

To limit disturbance within demarcated work area

Activity

Site clearance and establishing pile foundations

Potential impacts

- Ground clearance

- Bare and exposed areas

- Erosion

- Weed establishment

Possible causes

- Long exposure of disturbed areas

- Lack of rehabilitation

- Lack of adequate rehabilitation funds

Risks

Loss of soil

Reduction in land potential

Formation of gullies

Risk to powerline structures

Safety issue

Mitigation

- Identify and demarcate eroding areas

- Towers are to be placed outside these areas

- No driving is to take place along eroding area

- The local department of agriculture should be consulted to provide solutions to abate the widening of exiting gullies

- Implementation of the measures is to be monitored to ensure efficiency and stability

Targets

- No damage to existing erosion

- No towers along eroding areas

- Provision of conservation and engineering solutions to halt and/or retard widening gullies

Responsible

ECO, project coordinator, project engineer

Activity/Aspect

Pouring of cement

Cement must be poured directly into the hole. Any spillage must be removed and disposed accordingly

Objective

To limit poring within designated areas

Activity

Pouring of premixed cement to establish pile foundations

Potential impacts

- Concrete spills

- Ground pollution

Possible causes

- Indiscriminate pouring

- Lack of removal of accidental concrete spills

- Spilling cement along areas adjacent to the designated hole area

Risks

- Non-compliance

- Liability

Mitigation

- Only premixed cement is to be used to establish pile foundations

- No mixing of cement on site

- Any accidental spills should be removed off site and disposed at registered landfill sites

- Areas around the established pile foundations need to be cleaned of rubble and litter

Targets

- No concrete waste lying around

Responsible

ECO, project coordinator

Activity/Aspect

Construction of substation

The site work area is to be demarcated and fenced with a temporary fence. The perimeter fence will prevent livestock movement within the substation site work area. Security must be assigned to the site to protect construction material and prevent vandalism and site disturbance

Construction Activities:

- Soil excavation

- Site levelling

- Construction of banks

- Storage of soil

- Site compaction

- Erection of substation

- Construction of cut-off and vee-drains and outlet drains

- Construction of access road

- Application of approved herbicides on the high voltage site

Substation site

- The site has a slight slope towards the river

- Cutting and filling will be minimal

Erosion at substation

- No erosion was observed at both substations

- Soil are erodible care must be taken to limit disturbance within demarcated areas

- Construction is to be done during dry weather

- All bare areas to be demarcated

- No bare areas are to be exposed for long periods of time

- Department of Agriculture soil conservation is to be provide recommendations on erosion

Activity/Aspect

Demarcation of the site area, substation and access road

List of Associated Impacts

- Limits construction activity within site footprint thus minimise unnecessary damage

- Limits work within authorised area thus preventing illegal encroachment of neighbouring properties

- Creation of temporary access road outside the substation site

- Degradation of temporary access roads during heavy rains due to heavy vehicles

- Creation of passing bays for heavy vehicles along small local roads to allow passage of on-coming traffic

- Uncontrolled smoking areas leading to spread of fire

Possible Causes

- Compliance to legal requirements for site acquisition

- Elimination of ambiguity with regards to site boundaries thus containing construction work

- Staff ignorant of potential impacts and how they can be managed are likely to cause damage, pollution and environmental degradation

Risks

- Damage to neighbouring properties

- Damage to sensitive neighbouring properties and landuse thus increasing rehabilitation and compensation costs

- Bad relations with neighbours

Mitigation

- Site footprint to be demarcated using construction tape and subsequent erection of perimeter fence where applicable

- Substation boundaries are to be established and demarcated. All activities are to be conducted within Eskom boundaries. Construction activities that cause disturbance outside substation boundaries should be stopped and the disturbed area rehabilitated accordingly

- Activities outside site area such as movement of construction vehicles be limited to designated area

- All neighbouring natural areas such as bush, open grasslands, drainage lines, grazing areas etc are to be demarcated and protected from disturbance

- All disturbed areas are to be rehabilitated accordingly

- Before commencing with any work, all staff members will be appropriately briefed about the EMP and relevant occupational health and safety issues

- Educating staff about the content of the EMP will help with understanding all potential environmental risks, consequences of such risks and provide knowledge on how environmental risks can be avoided, minimised, prevented and controlled

- Rehabilitate temporary access roads to a better and stable state

- Rehabilitation of local roads to a better or stable state

- Rehabilitation of passing bays to the satisfaction of the affected landowner(s)

Targets

- No area outside work is significantly disturbed by work activities

- Where disturbed such as local roads, rehabilitation plan should be in place and implemented

- All staff to take part in environmental risk assessment

- Complete fencing of site area where applicable

- All work area marked with safety tape

- Communication of safety with neighbours where possible

Responsible

Project manager

Activity/Aspect

Excavation and storage of topsoil

Topsoil is needed for establishing grass cover along bare areas as a result of clearance of ground cover. Subsoil lack nutrients needed for growth

Objective

To prevent loss of topsoil

Potential impacts

- Disturbance of ground cover

- Movement of soil

- Loss/wash-away/blown away of loose soil under high winds and heavy rains especially stored topsoil and soil on substation banks

- Mixing of topsoil with subsoil

- Illegal dumping of excess subsoil and topsoil

Possible causes

- Lack of bund walls or sumps to contain wash-away soil before re-application

- Lack of separation of topsoil from subsoil

- Construction during rainy season

Mitigation

- It is highly recommended that excavations must not stand open longer period of times without proper security preferably they must be closed as soon as possible

- Materials needed to close excavated holes need to be readily available to prevent them remaining open for long periods

- Excavations must be marked with tape to clearly demarcate the area and warn against access

- Removed soil is to be used to backfill areas where required

- Deficiency of backfill material will not be made up by excavating illegally

- Imported backfill material should be purchased from legally registered merchandise

- Excess soil resulting from levelling activities of the work area should be stored in low heaps either on the access road or already disturbed area

- Topsoil is to be stripped together with grass / groundcover from all areas where permanent or temporary structures are located, construction related activities occur, and access roads are to be constructed

- Topsoil is to stored separately from subsoil and construction rubble

- Topsoil is to be placed away from vehicle movement

- Bund walls made of earth are to be constructed around or along topsoil mounds to help contain wash-away soil during heavy rains

- Topsoil is to be watered to help sink it and prevent being blown away

- Emergency preparedness for addressing potential loss of topsoil

- Used of crushed stones and soil binding material on access roads to control potential erosion

- Substation banks should receive proper compaction to prevent erosion but should allow penetration by roots

- Substation banks are to be watered lightly to help the soil to sink and bind

Targets

- Zero loss of topsoil

- Separation of topsoil from subsoil and/or rubble

- Existence of bund walls

- Existence of an emergency preparedness in case of heavy rains

Responsible

Project manager and project supervisor

Activity/Aspect

Dust Control

Substation site is not far from nearby houses. Dust can be carried over to the houses. Disturbed areas need to be watered during windy and dusty days

Objective

To control occurrence of dust along construction site

Potential impacts

- Air pollution

- Dust entering neighbouring properties

- Poor visibility

- Health implications

Possible causes

- Dust resulting from earthworks, excavations, disturbed bare areas such as substation banks and other bare areas

- Movement of vehicles along local gravel road and access road and around/within construction site

Risks

- Complaints from affected individuals

- Compensation for damage caused by dust

- Complaints on experiencing breathing problems

- Accidents due to poor visibility

- Stopping of work

- Loss of work time

Mitigation

- Water must be applied when necessary on areas where dust may occur

- Dust levels on site must be kept low at all times

- Water to control dust must be available when needed

- Permission to extract water from nearby streams must be requested from Department of Water Affairs (DWA)

- Use of crushed stones and soil binding material on access roads to minimise dust

Targets

Zero dust levels

No complaints about dust problems from neighbouring community

Responsible

Project manager and project supervisor

Activity/Aspect

Storage and use of chemicals

Office site plan should include areas designated for storing chemicals

Objective

To prevent accidental spillage and containment of spillage and to contain accidental fire

Activity

Storage and use of chemicals and other hazardous substances

Potential impacts

- Pollution of soil and water courses from leaked chemicals

- Killing of fish and other water species

- Pollution of water used by humans

- Sickness from drinking polluted water

Possible causes

- Lack of proper planning for storage of chemicals on site

- Use of leaking material to contain liquids

Risks

- Fines and litigation

- Damage to site and neighbouring

- Occurrence of fire

Mitigation

- All corrective plans are to be available for dealing with accidental spills

- The contractor shall comply with regulations of the Water Act (Act 36 of 1998), the Hazardous Substance Act (Act 115 of 1973) and the Environmental Conservation Act ( Act 73 of 1989)

- Repair all leaks of hydrocarbons or chemicals as soon as possible

- Do not allow sumps receiving oil or oily water to overflow

- Do not discharge oil or contaminants into storm water system

- Use bio-products for spills on soil, shake, water and tar

- Only Outpace, Outpace Super, and Roundup are to be used for controlling on site. Garlon is recommended as an additional herbicide to be applied on trees such as blue gum, wattle and bugweed. Non recommended herbicides are not to be used on site

- Herbicides not recommended by Eskom are not to be applied on the substation site, the HV yard. The need to use other herbicides is to be discussed with Eskom’s environmental officials at New Germany

- Contractors applying herbicides should be registered applicators Contractors are to use the recommended herbicides only

- After placing crushed stone on the HV yard and access road, these areas are to be treated with OUTPACE 100 GR (or OUTPACE SUPER) and after 6-7 months the treatment needs to be repeated

- A granular spread is recommended so that the dosing is not excessive and infects surrounding areas. A glyphosate-based herbicide such as ROUNDUP or NOMIX should be used to treat emergents prior and subsequent to the application of Outpace, or alternatively OUTPACE SUPER (which contains glyphosate as well) may be used

- All hazardous materials, petrochemicals, oils will be stored in a secured, appointed area that is fenced and has restricted entry

- Storage of hazardous products shall only take place using suitable containers

- Hazard signs indicating the nature of the stored materials shall be displayed on the storage facility or containment structure

- Fuel should be stored in a secure area in appropriate containers according to safety procedures

- Gas welding cylinders and LPG cylinders should be stored in a secure, well-ventilated area

- Sufficient fire fighting equipment must be provided in the event of an accident and strictly no smoking should be allowed where fuel is stored and used according to safety procedures

Targets

- No diesel or noxious chemicals, cement, etc. is to be spilt or left on site

- Zero seepage of herbicide to neighbouring properties

- Zero fire accident on site

Responsible

Project manager, ECO

Activity/Aspect

Pouring of cement

Cement is poured in holes to establish tower foundations. Only premixed cement is to be used

Objective

To limit poring within designated areas

Activity

Pouring of premixed cement to establish pile foundations

Potential impacts

- Concrete spills

- Ground pollution

Possible causes

- Indiscriminate pouring

- Lack of removal of accidental concrete spills

- Spilling cement along areas adjacent to the designated hole area

Risks

- Non-compliance

- Liability

Mitigation

- Only premixed cement is to be used to establish pile foundations

- Stability of the hole must be determined before pouring of cement

- Holes must be clean of debris and general waste before pouring of cement

- No mixing of cement on site

- Any accidental spills should be removed off site and disposed at registered landfill sites

- Areas around the established pile foundations need to be cleaned of rubble and litter

Targets

- No concrete waste lying around

Responsible

ECO, project coordinator

6. PREVENTING EROSION

Erosion prevalent in the area occurs predominantly below the west ridge and extends to drainage lines that join Tendeka River. Gullies occur along the riparian area and are caused and exacerbated by floods and livestock trampling and grazing along the riparian area. Areas where no erosion occurs are covered by grass.

6.1 	Objective

To prevent potential erosion as result of construction activities

Activity/Aspect

Loss of soil and formation of gullies

Potential impacts

- Increased water temperatures and decreased dissolved oxygen

- The loss of shade exposes soils to drying out by wind and sunlight and reduces the water storage capacity of the riparian area

- Loss of riparian vegetation causes river bank erosion

- Eroding banks contribute to sedimentation and lead to a wide shallow stream with little habitat value

- These factors result in significant reductions in aquatic stream life

Possible causes

- Livestock grazing and trampling along riparian area

- Reduction of porosity

- Decrease in permeability

- Decreases infiltration

- Increased run-off

- Erosion

- Water seepage along slopes under-cutting the soil

- Poor root density

- Rapid floods along river banks

Mitigation

- Locate towers outside gullies

- Span across riparian areas and rivers

- Position towers on or above upper terraces of riparian areas

- Protect or establish native shrubs, trees, or other vegetation along streams to help prevent bank erosion, trap sediment and filter other pollutants

- Manage livestock grazing in riparian zones to avoid damage to existing plants

- Fencing and re-vegetating riparian zones reduces erosion

- Involve Regional Department of Agriculture Soil Conservation to provide recommendations on effective methods that would prevent expansion of gullies

Targets

- No initiation of erosion from exposed denuded areas as a result of construction

- No towers located within riparian areas

- No towers near gullies

- Getting recommendations from Department of Agriculture on possible methods to prevent expansion of erosion

Responsible

Project manager, ECO, EEO

Table2 Summary of impacts and mitigation measures

| ACTIVITY | DESCRIPTION | ASSOCIATED IMPACTS | MITIGATION |
| --- | --- | --- | --- |
| Clearing of grass | Removal of any vegetation and compensation for loss, if applicable. A temporary activity | None |  |
| Erection of perimeter fence | To prevent livestock from trampling a construction site | Livestock trampling and can also have accidents Theft | Erect a temporary perimeter fence with a gate |
| Indication and clearing fire-belt strips | Clearing of a demarcated area around the substation perimeter | Erosion if firebelt area is denuded of vegetation | Grassing |
| Indicating and marking access road | Placing of pegs along the width and length of access road | None | Pegs must be visible |
| Soil excavation | According to site design plan to achieve the desired site level | Loss of soil | Impact can be prevented if done outside rainy season |
| Site levelling | Straightening and flattening of site | None |  |
| Construction of banks | According to design size | Loss of soil | Grass banks with a suitable grass sods made of indigenous material |
| Storage of soil | Topsoil and sub-soil for later use | Loss of soil if soil heap not protected from run-off | Separation of topsoil from sub-soil Placing of stones around soil heap |
| Site compaction | To achieve desired strength | Heavy compaction Water-logged soils Poor drainage | Compaction that allows filtration of water |
| Erection of substation | Building of all substaion components | None |  |
| Construction of cut-off and vee-drains and outlet drains | To chanell run-off away from substation high voltage (HV) site | Scouring of banks and siltation of HV yard Protection of equipment from malfunctioning | Effective draining slope and grassing of drains Placing of dissipators on drain outlet |
| Construction of access road | According to design specifications | Erosion | Access road must be able to channel run-off away from the access road Road edges must be grassed |
| Application of approved herbicides on the high voltage site | To prevent growth of vegetation on the HV yard | Damage of non-target vegetation A legal contravention | Use of approved herbicides at correct rates Applied by registered applicator |
| Stabilisation of banks | Grassing of banks to prevent erosion | Erosion | Depending on the slope either planting of grass sods or grass mixture |
| Removal of construction rubble and scrap metal | Removal of waste | Unwanted waste | Proper disposal and transportation to registered waste sites |
| Grassing of bare areas | Planting of vegetation to prevent scouring and erosion and growth of weeds Beautify the site | Erosion | Planting suitable grass seed mixture made up of indigenous grass seeds |
| Securing the substation site for access only for authorised persons | For safety purpose | Exposure to high voltage live equipment | Preventing the public from accidents at construction site |
| Application of herbicides | Only Eskom approved herbicides to be applied | Groundwater pollution Damage to non-target vegetation Pollution of nearby water cause Liability | A registered PCO should apply the herbicides |
| Preventing erosion | Stabilising all exposed areas | Loss of soil Land degradation Loss of habitat Sedimentation of rivers and streams Loss of aquatic life | Planting of bare areas with a suitable grass misture Managing livestock along riparian areas Fencing of riparian areas |

7. REHABILITATION OF SUBSTATION SITE AND BARE AREAS

Grassing of bare areas around the substation will prevent potential erosion and collapse of substation banks as a result of erosion. It will also prevent invasion of weeds.

- All disturbed areas must be identified and marked

- Rehabilitation of all disturbed areas must start once construction have ceased in the areas earmarked for rehabilitation

- Where topsoil is lacking it must be reapplied at 300mm layer

- 50mm of the top must be loosened before topsoil is applied

- The top layer of the topsoil is to be mixed with mulch to improve organic matter and nutrients in the topsoil

- If mulch is not available it must be imported

7.1 Use of grassing expert

Contractor shall make use of experienced grassing expert who has knowledge of indigenous vegetation.

7.2 Topsoil stripping

Before the topsoil is removed the aboveground grass cover must be removed with its roots and placed separately. This grass cover must not be mixed with rubble or topsoil it must be protected. After stripping topsoil the stockpiled grass cover is to be placed on top of the topsoil mound. This indigenous grass cover contains grass seed which will provide indigenous material that assist with re-establishment of the indigenous grasses that naturally occur in the area. Covering of stockpiled topsoil prevents potential erosion against wind and heavy rains. Insufficient grass covering material can be supplement by harvesting of indigenous grass sods from the surrounding area. These compensating grass sods are to be harvested sporadically to prevent concentrated harvesting thus leading to creation of significant bare areas. Harvesting should be done along open grasslands. Harvesting from private properties should not happen.

Topsoil is to be removed from all areas that are to be utilised during the construction period, where permanent structures are to be erected. Stripping of topsoil during dry conditions prevents compaction and later erosion.

7.3 Soil stockpiling

- Stripped topsoil shall be stockpiled in areas that will not be disturbed

- Soil stockpile mounds shall be lower than 2.5m

- Stockpiled topsoil should not be kept for longer periods least it loses its vitality and nutrients

- The slopes of soil stockpiles shall not be steeper than 1 vertical to 2.5 horizontal to prevent sheer erosion

- Topsoil stockpiles shall be clearly demarcated

- Soil stockpiles shall be prevented from any type of pollution, they shall be stored in areas away from contamination

- Soil stockpiles should not be stored along gullies or natural drains

7.4 Bank Retention

On closer inspection grass along the site appears overgrazed and it grows in bunches. Grass within the perimeter fence of the existing adjacent substation is made up of packed and well maintained kikuyu grass. Substation banks are to be grassed using fresh grass sods purchased from local nursery. A temporary perimeter fence is recommended to be installed to keep the grazing livestock out of the newly grassed banks and cut-off drains.

7.5 Grassing

Contractor shall use an experienced grassing contractor to design the grassing specification of the substation banks. The grass shall have been grown specifically for sod purposes. The grass specification shall include:

- Collection and harvesting of the grass sods

- Quality and type of grass sods to be used

- Type and quantity of fertiliser to be used

Grass sods shall be harvested, delivered and planted within 36 hours unless otherwise authorized by the experienced grassing expert. Grass sods shall be free from noxious weeds and diseases and shall contain a minimum of 30 mm soil. Period required to obtain good grass cover is to be specified by the experienced grassing contractor.

7.6 Seeding

NB: Seeding is recommended for areas where there is no proper fitting of sods. The recommended seed mixture is to be made up of indigenous seed mixture under the guidance of contracted grassing company. Only fresh certified seed mixture must be purchased from local nursery

7.7 After Planting Care

- The Contractor shall protect newly seeded/planted areas against undue traffic and/or other disturbances throughout the contract.

- Maintenance shall include continual repair of damage caused by erosion or any other causes.

- Upkeep of acceptable grass cover must be by reseeding or sodding when necessary.

- Watering should be done until proper growth has established.

8. COMPLETION OF WORK

When construction has been completed an audit of the area is necessary to ensure that all bare areas become grassed and construction and general waste around the work area has been removed.

Objective

To clear the site of all litter, rubble and dealing with pollution of soil and vegetation cover

8.1 	Mitigation

- After completing construction the contractor shall clear and remove from the site all construction waste such as empty containers, surplus materials, temporary foundations, temporary plumbing and other fixtures, rubbish and temporary work of every kind

- Areas thus cleared shall be graded and scarified and rehabilitated according to rehabilitation specification

9. OPERATION AND MAINTENANCE

Operational stage includes the energising of the powerline and substation and supply of electricity to customers. At operational stage the infrastructure is intact and operating

Objective

To prevent erosion, spread of fire rubbish lying around becoming a fire hazard

Activity/Aspect

Ground maintenance and effective safety

Potential impacts

- Eroded soil and silt landing on HV yard affecting quality of supply

- Waste lying around

- Tall grass a safety hazard

- Silted drains leading to improper channelling of run-off

- Poor grass cover around the substation yard

- Lack of grounds maintenance

- Lack of firebreaks

- Weed invasion

Possible causes

- Poor ground cover

- Poor ground maintenance

Risks

- Loss of topsoil

- Loss of supply

- Accidents

- Spread of fire

- Costs involved in importing soil to compensate for lost topsoil

Mitigation

- Planted areas need to be monitored for good cover

- Grass is to be cut short to prevent spread of fire and hiding of snakes

- Weeds are to be removed during cutting of grass

- All waste is to be removed from live substation yard up to the perimeter fence and areas around the perimeter fence

- Maintenance of suitable firebelts to prevent spread of fire

- Ground maintenance also prevents fire and hiding of snakes

- All areas where safety has been compromise need to be fixed

- Access road is to be monitored for good status and proper compaction

- Poor road condition must be corrected

- Silted drains are to be excavated and sloped accordingly

- Dissipaters at the end of drains must be maintained in good conditions to prevent scouring

- Substation site must be clean at all times

- Local Eskom office must report on any evidence of animal interaction and fatality

- Ablution facilities at the control room must be checked to be in good condition

- All safety signs along the substation must be checked to be in good order and replaced tarnished signs

- All complaints from neighbouring residents must be recorded and attended to

10. DECOMMISSIONING AND CLOSURE

Eskom assets when decommissioned they can lie idling for some time until a decision is made to remove them. Eskom still own the servitude and substation site after closure. It is Eskom’s imperative to monitor servitude or site encroachment. Servitudes and substation sites are kept as private property by Eskom for future use.

Objective:

To maintain a clean and sustainable environment for generations to come

Activity/Aspect

Removal of powerline and substation infrastructure

Potential impacts

- Pile foundations left behind

- Recyclable palisade fence around substation

- Idle substation buildings

- Material theft and vandalism

- Piles of crushed cement

- Waste lying around

- Hazard structural material

- Structural recyclable waste

- Weed invasion

- Bare and eroding areas

- Invasion of Eskom servitude or substation site

Possible causes

- Lack of decision about pile foundations and substation buildings

- Lack of removing waste from site and recycling it

- Poor removal of waste

Risks

- Vandalism and theft

- Loss of Eskom property rights

Mitigation

- All structural material to be stripped, removed from site and recycled

- Any empty pile foundations holes need to be refilled and grassed

- Underground electrical material beneath the high voltage substation need to be dug out, recycled and disposed

- The open substation HV yard hole needs to be refilled and grassed.

- General waste to be disposed at registered landfill sites

- Rehabilitating of bare areas around substation site is mandatory

- Complete removal of pile foundation

- Removal of all hazardous structural material

- Liaising with local authority about protection of Eskom property and servitude

- Weeds on site to be removed

Target

Complete removal of all structural material from site

Giving of some structural waste to locals upon approval by Eskom

Responsible

ECO, Project Coordinator

APPENDIX A

HERBICIDE APPLICATION PROCEDURE

|  |  | DISTRIBUTION–EASTERN REGION #### TECHNOLOGY AND QUALITY #### Engineering Instruction |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| REF NO: | EI-001-SSL |  | DATE: | 26-05-2006 | REVIEW: | 2008 | REV: | 2 |
| COMPILED BY: | TROY GOVENDER |  |  |  | APPROVED BY: |  |  |  |
| TITLE: | RECOMMENDED HERBICIDES |  |  |  |  |  |  |  |

RECOMMENDED HERBICIDES

The document below lists the herbicides commonly available that are possibly used at present, by Eskom personnel in the field.  From those listed, there are effectively only a few types that have acceptable environmental indices and are therefore recommended for use in the construction and maintenance of Eskom’s substations.

RECOMMENDED HERBICIDES FOR THE USE IN ESKOM SUBSTATIONS AND SECURITY FENCES

THIS INSTRUCTION MUST BE USED TOGETHER WITH THE HERBICIDE GUIDELINES

Read the herbicide guidelines and Eskom’s corporate strategy on herbicide management policy

ESKPBAAD4

Your first responsibility

Ensure that there will be no effect on plants outside the treated area - either immediately or in the future

STEP 1: Answer these questions.....

|  |  | YES | NO |
| --- | --- | --- | --- |
| 1. SUBSTAIONS | Does it have a drainage system? |  |  |
| 2. HV-YARD | On sloping ground? |  |  |
| 3. SECURITY FENCES | Or sandy soils? |  |  |
|  | Any adjacent crops, forestry, grazing? |  |  |

STEP 2: Choose your herbicide from these lists

|  | If you answered “yes” to ANY question you have a potentially dangerous situation and may ONLY use the following herbicides: |  |  |  |  | If all answers are “no” you may use any of the herbicides in the LEFT BLOCK, plus the following: |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| FOR EXISTING WEEDS |  | TO PREVENT WEED INFESTATION (control period up to 1 season) |  |  | GRANULAR (DRY) |  | LIQUID SPRAY APPLICATION |  |
|  |  | GRANULAR (DRY) |  |  | Atlacide |  | Amicide |  |
| Roundup Muster |  | Outpace 100GR |  |  | Bromacil 10GR Slam! GR Hydi G |  | Arsenal Bacprop S |  |
| Sting |  | LIQUID |  |  | Orbit |  | Triacide “S” |  |
| Tumbleweed Mamba Cobra |  | Outpace Flowable Simatop Weed Master S |  |  | Spike 5G |  | Tricate “S” Railroad SC |  |
| EXISTING WEED Outpace Super |  | & WEED PREVENTION or Edge |  |  |  |  |  |  |

STEP 3: Apply a safe herbicide in a safe manner

N.B. TO REDUCE ENVIRONMENTAL HAZARDS SUCH AS DUST, EROSION, and FIRE ETC., APPLY Vegetation Management (Species Switch) USING Outpace Super or Edge SC inside:

- grassed security fences

- properties between HV-yard and security fences

- other grassed areas

ANY HERBICIDES NOT LISTED, CONTACT YOUR ENVIRONMENTAL REPRESENTATIVE

Keep a Herbicide Usage Register for you are responsible and accountable for the correct use of HERBICIDES in your area

Furthermore, the purchase, use and storage of herbicides must comply with:

- Fertilizers, Farm Feeds, Agricultural Remedies and Stock Remedies Act, No 36 of 1947.   In terms of this Act, an applicator must be registered, with the relevant certification and training.

2. Eskom Policy on Herbicide Management, ESKPBAAD4

3. Eskom Standard on the Safe Use of Pesticides and Herbicides, ESKASAAL0.

Although it is envisaged that more frequent applications of these herbicides will have to be made to control alien vegetation, this is necessary to minimise long-term ecological damage to the natural environment, and reduce the concomitant legal, financial and publicity risks associated with environmental damage.

AUTHOR : 					RECOMMENDED BY :

TROY GOVENDER				RIAZ ASMAL

SENIOR ENVIRONMENTAL ADVISOR		TECHNOLOGY AND QUALITY MANAGER

SUPPORTED BY :				APPROVED BY :

KURT DEDEKIND				PETER SEBOLA

FIELD SERVICES MANAGER			ENGINEERING MANAGER

(a)	Herbicide Application

This refers to those areas to be put under stone in the substation (live) yard, and the entrance road.

After stoning it is recommended that the area as described must be treated with OUTPACE 100 GR (or OUTPACE SUPER) and after 6-7 months the treatment needs to be repeated.

A granular spreader is recommended so that the dosing is not excessive and infects surrounding areas. Rainfall is necessary to leach the herbicide into the soil before action can take place. Rainfall after application will give best results.

Outpace is applied dry – thus no water is required for mixing. Apply by hand or by using a granular applicator. Soil type should be determined so that the correct and effective dosage can be applied in order to obtain long lasting results.

Application of herbicide to non-target areas will result increase rehabilitation costs as a result of the death of newly planted grass, soil sterilisation for many months and the resultant soil erosion problems.

As a guide, 80kg to 200kg per hectare is used depending on the soil type and rainfall. For small areas use 1.2kg per 100m2 for control of annual grasses and broadleaf weeds.

# (b)	When to Use Outpace 100 GR

OUTPACE 100 GR is a special blend of low-leaching herbicides at low environmental risks. OUTPACE 100 GR is used as a maintenance treatment applied before the emergence of the weeds. Weeds should initially be removed by mechanical means.

## (c) 	Treatment of Weeds Prior and After Application of Outpace 100 Gr

A glyphosate based herbicide such as ROUNDUP or NOMIX should be used to treat emergents prior and subsequent to the application of Outpace, or alternatively OUTPACE SUPER (which contains glyphosate as well) may be used. Outpace Super should be applied at 25 l/ha.

GLYPHOSATE deactivates on contact with the ground and water. However, it is highly effective on green leaves and green stems (it reacts with the green chlorophyll producing parts of the plant).  Results might not be immediately noticeable but occur over a period of a few weeks therefore respraying is an unnecessary wastage.  The best form of glyphosate containing herbicide is currently being marketed by NOMIX; being safe to handle.  It is further insoluble within an hour of application to plants.  Other products require the absence of rain, for at least 4 hours, to obtain best results.

# (d) 	Precautions

- Read the entire label before using the product and ensure that the requirements for its correct use are observed.

- Ensure that persons assigned to apply the products are adequately trained.

- Do not apply to streams or dams.

- Do not apply to areas designated for future cultivation.

- Although OUTPACE is low in toxicity to humans, use a dust mask to prevent inhaling fine particles when applying by hand or when chest carried applicators are used.

- Use gloves and protective clothing when handling the product, and prevent eye contamination.

- Do not use near desirable plants or trees, or in areas where their roots may extend, or in location where chemical may be washed or moved into contact with their roots.

- To avoid injury to desirable plants, application equipment used for application must be thoroughly washed and rinsed.

Refer to:-

GUIDELINES FOR WEED ERADICATION IN ESKOM SUBSTATIONS Ref. No: TRR S92/034.

GENERAL CONDITIONS OF CONTRACT FOR CHEMICAL / MECHANICAL CONTROL OF VEGETATION.  Ref.: Rev 3, Feb., 1992.

A more precise description of the attributes of the herbicides mentioned is discussed in these documents.

The above must be complied with and this information disseminated to those directly involved with herbicide application, or the supervision of herbicide contractors.